Multi-Anvil High Pressure Cell Assembly Development

2018 COMPRES Annual Report

November 2017 – October 2018

Prepared by Kurt Leinenweber, Eyring Materials Center, Arizona State University

Overview

The COMPRES Multi-Anvil Cell Assembly facility provides cell assemblies to all interested users of multi-anvils who are members of COMPRES.  This includes those with Kawai type multi-anvil presses in their laboratories, as well as beam lines that have Kawai type presses, DIA presses, and Paris-Edinburgh presses.  The pressure media can be octahedral, cubic, or other shapes depending on the design of the apparatus.  In 2017-2018 we served many laboratories around the world, either working with them directly, or in the case of laboratories in China, mostly working through intermediaries but still communicating directly with the laboratories on technical issues.

The COMPRES Cell Assembly Development Project has been working on multi-anvil cell assemblies in a community-wide effort since the original beginnings of COMPRES.  This year has seen the further maturing of this project with new developments, a new level of popularity for the standard cell assemblies, some challenges in meeting the new demand, and a resulting resolution of the challenges due to “redevelopment” of some of the primary developments from before.

Challenges

We’ll start by describing the various unexpected challenges to the project this year.  Some of these come from the COMPRES community itself, meaning all of those who rely on the cell assembly project to keep research programs going, while others have come from new difficulties in procuring the parts of the cell assemblies in a timely fashion.

From the COMPRES community, we had challenges such as short lead times on assembly requests, though not through any fault of the users.  The beam lines are locked into a very rapid schedule in which beam time requests are evaluated each cycle, and provisions for each request are not requested until after the beam time is allocated.  This had not caused many issued in the past, but with more beam lines needing materials (GSECARS,Sector 6, NSLS II and the Australian beam line have all been active in our project this year), and with requests from both beamlines and individual users, there has been more of an issue this year.  We have found ourselves scrambling on several occasions to fill orders in time for allocated beam time.

In order to address this, we need to increase the stock of parts for beam line assemblies that will be maintained for future needs.  The GSECARS beamline, for instance, has a number of standard assemblies and in the beginning they were being supplied in small numbers for each beamtime.  But, this led to shortages when our machine shop lead time became longer than 2 months.  2 months seems to be the maximum warning for new beam time needs.  It seems worth the investment to lay in a stock of parts for anticipated future beam times when such parts are standard and expected to be needed for the foreseeable future.  There is some financial risk in this for COMPRES but it is small compared to the value of beam time that might be wasted because of a lack of cell assembly resources.

Parts such as crushable alumina sleeves, MgO plugs and rings, and graphite furnaces are common and are readily supplied in large numbers by our vendors (Machined Ceramics for the crushable alumina and MgO, and Mersen for the graphite sleeves).  The designs of these parts are specified by the beam line, at first were very unfamiliar to us but have now remained consistent over several beam cycles.  We have been stocking up on some parts for GSECARS but there were some assemblies left out of the mix because of small initial demand, and yet those are the ones that have caused problems recently.  So we are now laying in a stock of all of the beam line assemblies.  The only parts “made to order” will be boron nitride (BN), because it is air-sensitive and has a shelf life.  Later on even BN might be made in larger batches in advance, and kept in a sealed environment, if necessary.

A warning here is that some designs change.  We have to be careful about which items we decide to procure in larger quantities.  We are currently laying in a stock of parts for the GSECARS and Australian DIA assemblies which have been consistent over the last 2 years or so and for which users are happy with the designs.

The assemblies for the Sector 6 DIA press present special challenges that seem to repeat each cycle.  The tiny snug-fitting parts and very thin furnaces cannot be made by outside vendors, and thus always have to be made at the ASU shop.  The furnaces are so thin that they sometimes do not behave consistently from piece to piece.  We will continue to work on solving these challenges in collaboration with the beamline staff at Sector 6.

Challenges for the Standard “Offline” Assemblies

In the midst of further increasing demands – with several dozen orders per year that range in size from 5 to 500 assemblies per order – plus increased urgency to try to ship the assemblies overseas before new tariffs are levied against them – has coincided with the recent closing of our rhenium cutting factory (Directed Light in California) and the end of our supply of Cherry-O lanthanum chromite.  We therefore now have a very low stock for rhenium furnaces and lanthanum chromite parts, which are needed for several of the COMPRES designs.  We have identified a replacement grade of lanthanum chromite in Japan thanks to advice from Dan Frost, and have found a possible source for rhenium cutting in Germany thanks to Monika Koch-Müller, but we are still setting up the first test of that vendor.  KL did manage to locate the technician who used to cut the rhenium at Directed Light, who now deals in laser-cutting equipment and who has agreed to help us find a replacement vendor in the US.  If the rhenium and lanthanum chromite issues are solved soon, we can replenish the stocks and continue to keep up with the current, high demand for the COMPRES assemblies.

Re-developments

The maintenance of consistent assemblies when supply chains are cut off is a kind of development, but might be better named “redevelopment.”  It is something that was already developed, but must be developed again to match previous capabilities – or that has to be redeveloped to meet higher demand.  This has occupied a lot of our attention recently.

New developments

Forsterite thermal insulation has been successfully tested in several applications and we anticipate that it can be an inexpensive and x-ray transmitting replacement for zirconia.  We have been using it in specific instances (DIA pressure transmitting media, thermal insulating sleeves for multi-anvil assemblies) but have yet to try it on a large scale.  We will work with the supplier of this material to make large quantities of parts in the next year, now that it has passed initial testing.

A 9 mm DIA assembly for the Australian beam line was designed by Jeremy Wykes and trial batches were made at the ASU shop.  Over a few beam cycles, we have built up stocks of most of the parts as of fall 2018, and soon will only have to make the boron nitride sample containers fresh in each cycle.

For GSECARS, we have taken on the task of making parts for many predesigned assemblies (DIA and Paris-Edinburgh).  We took over the provision of these, and despite some close calls on deliveries, are on the way to having a steady and reliablesupply of these assemblies.

We also attempted to make some of the very tiny parts for the DELVE initiative of Yanbin Wang and Tony Yu.  A significant fraction of these parts, though, were not amenable to our current methods, though others were useful.  The main problems were with pyrophyllite parts that were too miniaturized to be made by our usual vendor (Bar-lo), and also miniscule graphite capsules that did not look very nice under a microscope.  We can already make tiny MgO and alumina plugs.  We have since found a company that is willing to make the tiny pyrophyllite parts, and we need to search for micro-machining companies that can do the graphite and other parts.  We also need to find a source for machining complex ceramic shapes for Paris-Edenborough cells and split octahedral assemblies used at the beam lines.  We hope to make progress with these items in the upcoming year.

The DIA assemblies for Sector 6 still have issues in fabrication.  We hope to make improvements to this in the upcoming year, with Haiyan Chen and Matthew Whitaker acting as critical users of the assembly attempts.  Also, some new designs that are within the capabilities of major vendors are under testing by Whitaker.

We have provided some trial pieces for the new multi-anvil beam line at NSLS II.  These entirely new, made-from scratch parts and pieces for an 8/3 assembly were made using designs that were discussed with D.J. Weidner and others at that new beam line.  These have been fabricated tested in two beam cycles and have been working well so far, and we have hopes that this will develop into a useful design for this new facility.

Scientific Highlights

This year’s highlights include the new electrical conductivity assemblies, with many measurements and results coming out in the last year.  The assemblies are available to interested users now.  Also, a new impedence spectrometer has been installed at GSECARS by Anne Pommier and we will be trying the first online experiments with it (including x-ray) in late November 2018.

Beamline Personnel

We have employed three undergraduate workers who have helped with the packing of the approximately 2300 assemblies that we provided this year.  Without the assistance of these workers, the supply would not be able to be maintained.  Kurt Leinenweber still does the primary ordering for the project, and does a quality check when packing each order.  The current undergraduates are Logan Leinbach, Devin Keating and Andrew Eubanks.  Andrew Eubanks in particular has experience with inventorying and has been a great help in keeping track of our inventories and supplies, which has been quite difficult in the past.

Beamline Operations

The requests for assemblies and parts are now handled through a service called iLab.  Each order received a number, and then we fill them in the order they are received, if possible.  However, we also respond to emergencies when necessary, which happens particularly often with beamlines, but also can happen in offline laboratories.  Recently, ASU obtained an inventory service in iLab as well, and all of our various assembly parts are in the process of being added to the inventory.  In the past 12 months we have received requests for 2343 assemblies as well as 63 carbide cubes and assorted individual parts.

Performance Metrics

The list of users and of assemblies is given here in tabular form.  The information is all taken from actual records in iLab that occurred in the 1-year period listed, and does not overlap with any information from previous reports.

Table 1: List of COMPRES cell assemblies and the number provided in a 12-month period (Nov. 1, 2017-Nov. 1, 2018).

| Size and Name | Type of heater | Max Pressure | Max Temperature | Number provided Nov 2016-Nov 2017 |
| --- | --- | --- | --- | --- |
| Standard 6-8 (Kawai type) assemblies |  |  |  |  |
| 8/3 | Rhenium (Re) | 25 GPa | 2319 C | 100 |
| 10/5 | Re | 20 GPa | 2319 C | 716 |
| 14/8 “G2” | Graphite box | 13 GPa | 1200 C | 714 |
| 14/8 “HT” | Re | 15 GPa | 2000 C | 85 |
| 14/8 “EC” | Re | 14 GPa | 2000 C | 34 |
| 18/12 | Graphite | 10 GPa | 1500 C | 359 |
| 6-8 Kawai type assemblies for in-situ XRD |  |  |  |  |
| 8/3 | Re/window | 25 GPa | 2000 C | 0 |
| 10/5 | Re/window | 21 GPa | 2000 C | 0 |
| 14/8 equatorial | Graphite | 15 GPa | 1200 C | 20 |
| DIA assemblies for in-situ XRD |  |  |  |  |
| Standard (S-DIA) sector 6 | Graphite | 8 GPa | 1200 C | 55 |
| Deformation (D-DIA) sector 6 | Graphite | 8 GPa | 1200 C | 65 |
| Ultrasonic U-DIA sector 6 |  | 8 GPa | 1200 C | 50 |
| Australia DIA | Graphite | 8 GPa | 1200 C | 25 |
| D-DIA 9, sector 6 | Graphite | 5 GPa | 1400 C | 25 |
| GSECARS DIA, PE cells | Graphite | 8 GPa | 1200 C | 75 |
| NSLS II trial cells | TiB2/BN | 25 GPa | 1500 C | 20 |
| TOTAL |  |  |  | 2343 |

Table 2: COMPRES Users of cell assemblies in iLab records in the period 2017-2018

| Name | Lab/PI | Institution |
| --- | --- | --- |
| Alexandra Tamerius | Freedman, Danna | Northwestern University |
| Alisha Clark | Clark, Alisha | Northwestern University |
| Anne Pommier | Pommier, Anne | University of California at San Diego |
| Bin Chen | Chen, Bin | University of Hawaii at Manoa |
| Brittany Kulka | Shim, Dan | Arizona State University |
| Caleb Holyoke | Holyoke, Caleb | University of Akron |
| Charles Lesher | Lesher, Charles | University of California, Davis |
| Denivelle, Melanie | Denivelle, Melanie | ARDOP Laboratory, Pessac, France |
| Donald Weidner | Weidner, Donald | Stony Brook University |
| Emil Stoyanov | Malik, Abds-Sami | Sandvik Hyperion (Industrial) |
| Eun Jeong Kim | Lee, Sung-Keun | Seoul National University |
| Gabriel Gwanmesia | Gwanmesia, Gabriel | Delaware State University |
| Guillame Morard | Morard, Guillame | CNRS |
| Haiyan Chen | Weidner, Donald | Stony Brook University |
| Jacqueline Tappan | Shim, Dan | Arizona State University |
| Jennifer Kung | Kung, Jennifer | National Cheng Kung University |
| Jeremy Wykes | Wykes, Jeremy | Macquarie University |
| Jing Yang | Fei, Yingwek | Carnegie Institution Geophysical Laboratory |
| Jonathan Dolinschi | Shim, Dan | Arizona State University |
| Kirill Cherednichenko | Solozhenko, Vladimir | Laboratoire des Sciences des Procédés et des Matériaux |
| Kurt Leinenweber | Leinenweber, Kurt | Arizona State University |
| Kyusei Tsuno | Dasgupta, Rajdeep | Rice University |
| Matthew Whitaker | Weidner, Donald | Stony Brook University |
| Paul Raterron | Raterron, Paul | Brown University |
| Pinwen Zhao | Zhao, Pinwen | Jilin University |
| Richard Triplett | Weidner, Donald | Stony Brook University |
| Robert P. Rapp | Rapp, Robert P. | Australian National University |
| Samantha Clarke | Freedman, Danna | Northwestern University |
| Steven Webb | Webb, Steven | BHGE Lab (Industrial) |
| Tony Yu | Wang, Yanbin | University of Chicago |
| Ulrich Haussermann | Haussermann, Ulrich | Stockholm University |
| Wenjun Yong | Secco, Rick | University of Western Ontario |
| Xiancheng Wang | Wang, Xiancheng | Institute of Physics, The Chinese Academy of Sciences |
| Yongtao Zou | Zou, Yongtao | Jilin University Superhard Materials |
| Yongzhae Lee | Lee, Yongzhae | Yonsei University, Kiorea |
| Yun-Yuan Chang | Hsieh, Wen-Pin | Academica Sinica, Taiwan |
| Yuegao Liu | Liu, Yuegao | Focus Magmatism and Giant Ore Deposits Lab, Xi'an Center of Geological Survey, China Geological Survey |
| Zhenmin Jing | Jing, Zhenmin | China University of Geosciences (Wuhan) |

Beamline Community Activities

Our main community activity is a back and forth communication about the cell assemblies.  Users send comments and describe experiences, or troubleshoot the assemblies with us.  We have made a lot of joint progress on the assemblies that would not be possible otherwise.

Kurt Leinenweber traveled to the Chinese Institute of Geosciences (CUG) and presented a talk on the cell assembly development project and on scientific results, in September 2018.  The trip was funded by CUG.

Budget

The budget is attached as an Excel Spreadsheet, in NSF format.  It includes part of the undergrad wages (the rest will be paid by the State of Arizona as part of our lab budget), and ½ month of salary for Kurt Leinenweber.  Materials and supplies are for trial batches of ceramics and parts for multi-anvil cell assembly testing in collaboration with participating laboratories and beamlines.

Publications

(2017)

Cheung, S. N. C.; Weidner, D. J.; Li, L.; Meredith, P. G.; Chen, H.; Whitaker, M. L.; Chen, X. (2017) Stress distribution during cold compression of a quartz aggregate using synchrotron X-ray diffraction: observed yielding, damage and grain crushing. Journal of Geophysical Research (Solid Earth), 122 (4) 2724-2735

Clarke, S.M.; Amsler, M.; Walsh, J.P.S.; Yu, T.; Wang, Y.; Meng, Y.; Jacobsen, S.D.; Wolverton, C.; Freedman, D.E. (2017)  Creating binary Cu-Bi compounds via high-pressure synthesis: a combined experimental and theoretical study.  Chemistry of Materials 29, 5276-5285.

Li, Z.; Li, J.; Lange, R.; Liu, J.; Militzer, B. (2017)  Determination of calcium carbonate and sodium carbonate melting curves up to Earth’s transition zone pressures with implications for the deep carbon cycle.  Earth and Planetary Science Letters 457, 395-402.

Lu, J.; Kou, Z.; Liu, T.; Yan, X., Liu, F., Ding, W.; Zhang, Q.; Zhang, L.; Liu, J.; He, D. (2017)  Submicron binderless polycrystalline diamond sintering under ultra-high pressure.  Diamond and Related Materials 77, 41-45.

Nemeth, P.; Leinenweber, K.; Ohfuji, H.; Groy, T.; Domanik, K.J.; Kovacs, I.J.; Kovaks, J.S.; Buseck, P.R. (2017)  Water-bearing, high-pressure Ca-silicates.  Earth and Planetary Science Letters 469, 148-155.

Nisr, C.; Leinenweber, K.; Prakapenka, V.; Prescher, C.; Tkachev, S. and Shim, S-H. (2017)  Phase transition and equation of state of dense hydrous silica up to 63 GPa.  J. Geophys. Res., 122.

Nisr, C.; Shim, S-H.; Leinenweber, K.; Chizmeshya, A. (2017)  Raman spectroscopy of water-rich stishovite and dense high-pressure silica up to 55 GPa.  American Mineralogist 102, 2180-2189.

Righter, K.; Guo, B.M.; Pando, K.A.; Danielson, L.; Ross, D.K.; Rahman, Z.; Keller, L.P.  (2017)  Phase equilibria of a low S and C lunar core: Implications for an early lunar dynamo and physical state of the current core.  Earth and Planetary Science Letters 463, 323-332.

Silber, R.E.; Secco, R.A.; Yong, W.J.  (2017)  Constant electrical resistivity of Ni along the melting boundary up to 9 GPa.  Journal of Geophysical Research 122, 5064-5081.

Spektor, K.; Wan, W.; Nedumkandathil, R.; Andersson, O.; Haussermann, U. (2017)  Crystallization of mesoporous silica SBA-15 in a high pressure hydrothermal environment.  High Pressure Research 37, 345-349.

Stagno, V.; Bindi, L.; Steinhardt, P.J.; Fei, Y.W.  Phase equilibria in the nominally Al65Cu23Fe12 system at 3, 5 and 21 GPa: implications for the quasicrystal-bearing Khatyrka meteorite.  Physics of the Earth and Planetary Interiors 271, 47-56.

Whitaker, M.L., Baldwin, K.J., and Huebsch, W.R. (2017) DIASCoPE: Directly Integrated Acoustic System Combined with Pressure Experiments – A new method for fast acoustic velocity measurements at high pressure. Reviews of Scientific Instruments. 88 (3) 034901

Yong, W.; Secco, R.A.  A simple system for low-temperature experiments in a large-volume multi-anvil press.  Review of Scientific Instruments 88, article number 106106.

Zhang, Y.; Jin, Z.; Griffin, W.L.; Wang, C.; Wu, Y. (2017)  High-pressure experiments provide insights into the Mantle Transition Zone history of chromitite in Tibetan ophiolites.  Earth and Planetary Science Letters 463, 151-158.

(2018)

Cheung, C.S.N.; Weidner, D.J.; Li, L.; Meredith, P.G. Chen, H.Y.; Whitaker, M. Chen, X.Y. (2018)  Stress distribution during cold compression of rocks and mineral aggregates using synchrotron-based x-ray diffraction.  JOVE – Journal of Visualized Experiments 135, article number e57555.

Knibbe, J.S.; Luginbuhl, S.M.; Stoevelaar, R.; van der Plas, W; van Harlingen, D.M.; Rai, N.; Steenstra, E.S. van de Geer, R.; van Westrenen, W. (2018)  Calibration of a multi-anvil high-pressure apparatus to simulate planetary interior conditions.  EPJ Techniques and Instrumentation 5, article number 5.

Pommier, A.; Leinenweber, K.D.  (2018)  Electrical cell assembly for reproducible conductivity experiments in the multi-anvil.  American Mineralogist 103, 1298-1305.