COMPRES Multi-Anvil Cell Assembly Development

2016 COMPRES Annual Report

November 2015 – October 2016

Prepared by Kurt Leinenweber

PI’s: Kurt Leinenweber, James A. Tyburczy, Thomas G. Sharp (Arizona State University)

Overview

Summarizing the facility, its capabilities, operations, achievements, etc.

The COMPRES Multi-Anvil Cell Assembly Development program, located at Arizona State University, is a full-service cell assembly workshop specializing in the development and supplying of working multi-anvil cell assemblies to multi-anvil laboratories around the world.  The program has developed a number of standardized high-pressure cell assemblies, listed in Table 1, that are in widespread use at various laboratories.  The many parts (approximately 10-20 different parts for each assembly) are made at the ASU machine shop, where two of the eight machinists in particular specialize in ceramics and other multi-anvil specialized tasks such as carbide truncation, and at outside ceramics grinding or milling companies.  Users can request standardized assemblies, but also specialized parts for their own needs and are now able to order the parts directly from our shop as well.  In the period from 10/2/2015 to 11/11/2016 (the entire period covered by our new online system iLab described below), we supplied 3483 of our standard assemblies to the community, with the breakdown listed in Table 1.  We also supplied many special parts to order, and truncated tungsten carbide cubes as needed by researchers.  We did not have to turn down any requests this year.  A new system adopted by ASU, iLab, allowed COMPRES researchers to request the materials online, and makes it much easier for us to keep track of the orders, to make sure the orders are processed in the correct sequence, and not to forget anyone.

Scientific Highlights

Brief discussion of a small number of featured studies illustrating recent successes from the beamline for this year. Encourage users to send in one-pagers to include in the appendices of your annual report.

In 2015 we worked with two high-pressure researchers at Sandvik Hyperion (formerly Diamond Innovations), Sami Malik and Emil Stoyanov, to develop a gauge for measuring high pressure and temperature.  We developed a quenchable solid solution that records the pressure at a given temperature, in the range 2-10 GPa and 1200-1500 degrees C.  The gauge, based on the lattice parameter of a (Ge,Si)O2 rutile phase that was in equilibrium with coesite, has proven to be quite sensitive and is in frequent use in the production facility at Sandvik, as well as being used to calibrate COMPRES assemblies.  (Gullikson et al., 2015; Leinenweber et al, 2015).

Another success was in the development of working electrical conductivity/impedence assemblies based on the COMPRES design.  The most successful design so far is based on the COMPRES 14/8 “HT” assembly and used the thermocouple as one electrical lead while introducing an additional wire as the other lead.  The measurements in Pommier et al. (2015a) and Pommier et al. (2015b) were based on this design.

Several of our assemblies have proven popular in the new laboratories in China and have allowed those laboratories to get set up and going quickly.  Several studies of geological problems have been ongoing in these laboratories.  These researchers have also done extensive characterization of the COMPRES assemblies, such as pressure calibrations and thermal gradient measurements, which will prove useful to the user community as a whole.  (Wu et al., 2015; Zhang et al., 2015; Zhang et al, 2016)

Beamline Personnel

Discuss management structure; current beamline staff; other key personnel, and sources of funding, including funding external to COMPRES (this is important to identify how the facility personnel leverage COMPRES support). Any changes in COMPRES-supported staff should be discussed, with CVs of new hires included as appendices.

Leveraging in this project is significant, including the use of the subsidized ASU Machine Shop which is operated by the College of Liberal Arts and Sciences (CLAS) at ASU; the multi-anvil facility is also run through CLAS, Kurt Leinenweber’s salary comes initially from the state of Arizona (but KL recovers part of it as described below), and we have also hired two undergraduate workers who help with the packing and shipping of materials for the project.  They are paid out of CLAS and their salary is recovered from overhead on the assemblies that are distributed through this project, as is about half of Kurt Leinenweber’s salary.  Much of the equipment used to test the assemblies (multi-anvils, and auxiliary equipment, x-ray diffraction facilities, electron microprobe, etc.) is run by CLAS.  COMPRES supports one month of Kurt Leinenweber’s salary to offset costs to the state associated with cell assembly development for the COMPRES community – this funding is transferred to the state account to offset the salary paid by the state for that month.  The sale of cell assemblies and other high-pressure related materials to users resulted in revenues of $475,797.04 according to iLab records, for the period 10/2/2015 to 11/11/2016 ( a little over a year).  This provides leveraging because it allows us to continue to replenish the assembly stocks and keep the project going, while using the COMPRES funding to pursue new developments.

Beamline Operations

Describe beamline operations in the past year. Include the amount of beamtime available; the relationship between COMPRES and the synchrotron facility regarding beamline operations; beamtime proposals and other beamline access; and other relevant discussion.

COMPRES requires also the following specific statistics for the past two years for each beamline:

Number of iLab inquiries received: 145

Number of iLab orders filled: 92

Total number of assemblies requested: 3483

Total number of assemblies granted: 3483

Total number of assemblies available: 3483

Oversubscription rate (= shifts requested / shifts available) 1.0

Number of visits by distinct research groups: 4

Number of unique users, categorized by affiliation (University, Gov’t Lab/agency, Private institution, or Industry) and by origin (USA, Canada, Europe, Asia, Other)

Total number of person-visits:  4

Number of undergraduate users:  2

Number of graduate student users:  4

Number of visits funded by each funding agency (NSF, DOD, DOE, Foreign/Other)

Please be clear about how shifts are counted, especially with regard to side stations and parasitic modes of operation. Details of the successful proposals, beamtime allocated, and total usage should be provided in an appendix.  See Tables.

Performance Metrics

This information has been requested by the NSF-IF program managers and is part of the reporting requirements written into our 5-year Cooperative Agreement with the NSF. As such, you are requested to supply this information on an annual basis. To make reporting easier and to obtain uniformity in the information we receive, two files are attached to be used as templates for the Performance Metrics reporting.

One spreadsheet is for user statistics for the past year. We require: PI name, country, funding source, and beamtime allocation. Please also provide:

General User Time for COMPRES: For higher-ranked proposals with beamtime allocated by the facility.

Contributing User Time: Allocated for COMPRES users who were not assigned GU time.

Total time for COMPRES users (there will usually be GU and CU COMPRES users).

The second template is for the upcoming run cycle (that is, beyond the current run cycle). These tables are hopefully self-explanatory. If there are any questions, please let me know.

Our main metric in this report is in the provision of assemblies to the members of the community, and the number of assemblies we have provided is significant.  We have kept the information about how many assemblies are ordered by each member confidential, because we have never obtained permission to share this information from the end users.  If this information is needed for the Performance Metrics, then we could send out an e-mail to all of them asking for permission to share the information – or COMPRES can provide us with assurance that it is OK for me to share the information without asking.  Also, we have not gathered information on any of their funding sources.

Beamline Community Activities

Any outreach activities, workshops hosted, media features highlighting beamline activity, safety improvements, etc. should be summarized. Other activities of the beamline scientists, for example related to their professional development, independent research, or other community activity, could be discussed here too.

Kurt Leinenweber participated in the COMPRES workshop on Large Volume Multi-Anvil acquisition held at the COMPRES 2015 annual meeting, organized by Dan Shim.  He has been active in the efforts to obtain such a press at Arizona State University and will also provide cell assembly help if a press is acquired by Jacobs in Houston (Lisa Danielson).

Beamline Development

COMPRES beamlines are typically in a state of continuous development. Discuss recent improvements, ongoing upgrades, and important problems or challenges to the facility.

We have had several upgrades to our facility that improve the cell assembly development capabilities.  We acquired an optical microscope with a camera attachment that will be used to document cell assemblies and to produce more instruction manuals for the assembly preparation.  We have improved our glassmaking capabilities and are providing SiO2-GeO2 glasses to the community for use in pressure calibration (so far it has been provided to 2 laboratories).  This is the inception of a plan to provide high-quallity pressure calibrants (SiO2-GeO2, Mg2SiO4, CaSiO3, FeSiO3, CaGeO3, MgSiO3, etc) in modest quantities free of charge to members of the community as fixed-point or sliding-scale pressure calibrants.

We will continue working with Yanbin Wang on development of 50 GPa capabilities for the multi-anvil.  The first task will be to try the standard COMPRES 8/3 assembly using 3 mm truncated ultrahard carbide cubes from Fujilloy.  Our project has already purchased some of these cubes 25.4 mm in size and they are on the way.  Yanbin has purchased some smaller cubes of the same truncation for the T-cup.  The capabilities will be tested at the beamline using COMPRES assemblies.

Planned Activities

Describe beamline improvements or other activities that are planned. For COMPRES facilities now moving their operations to a new beamline (at NSLS-II and/or APS), this should obviously include a detailed discussion of the transition plan, including a timeline of activities related to the move, and a description of how and where COMPRES-supported staff will be utilized during and after the transition.

We will acquire a high-fire furnace in order to make significant improvements in our firing capabilities for ceramic parts needed for this project, and also for creating pressure standards and other research materials for the community.  Currently we fire parts at 1000 C for 1 hour before providing them, in cases where such firing is appropriate.  However, for some cases such as stabilized zirconia sintering, or preparing MgO for electrical conductivity parts, higher temperatures such as 1400 C are required.  Also, for making many of the pressure calibrants, glasses and other materials often requested by members of the community, a high fire furnace is needed.

We wish to become more involved also in the encapsulation methods for various high-pressure samples.  We provide a few options for encapsulation, but the needs of the community are much greater because of the wide variety of samples treated under pressure.  We would like to work on development of a variety of ceramic capsules and metal capsules for all the standard COMPRES assemblies.

We are currently developing the DIA production cells for GSECARS and have made a number of trial cells not yet listed in our Table of standard assemblies.  When these are verified they will become part of the list and the statistics for the year.  We also have agreed to look into the possibility of making boron epoxy pressure media for the in-situ XRD community, something we have not been doing in the past.

Budget

In order to avoid confusion that might arise from creating new budget documents, this budget justification is copied verbatim from the COMPRES renewal proposal.  Also, the budget spreadsheet submitted to UNM for that proposal is provided as an attached document.

ARIZONA STATE UNIVERSITY BUDGET JUSTIFICATION

Senior Personnel

Dr. Kurt Leinenweber, PI for the project, will commit 1 calendar month of effort per year. He will be responsible for overseeing the development, testing and distribution of prototype multi-anvil cell assemblies to the research community as described in the project proposal.

Other Personnel

TBD, Undergraduate Student Hourly Worker

Salary is requested for one undergraduate student worker who will be hired for a total of 20hrs/week at $10/hr per year throughout the calendar year and 40hrs/week in the summer. The student will be trained in multi-anvil techniques and will help with the testing of the prototype assemblies for COMPRES.

Fringe Benefits

Arizona State University defines fringe benefits as direct costs, estimates benefits as a standard percent of salary applied uniformly to all types of sponsored activities, and charges benefits to sponsors in accordance with the Federally-negotiated rates in effect at the time salaries are incurred.  Benefit costs are expected to increase approximately 3% per year; the rates used in the proposal budget are based on the current Federally-negotiated Rate Agreement plus annual escalation for out years.

|  | Year 1 | Year 2 | Year 3 | Year 4 | Year 5 |
| --- | --- | --- | --- | --- | --- |
| Staff | 38.52% | 39.68% | 40.87% | 42.10% | 43.36% |
| Hourly Undergraduate Student | 0.62% | 0.64% | 0.66% | 0.68% | 0.70% |

Travel – Domestic

$3,500 is budgeted per year for travel to the GSECARS LVP beam line in Argonne, Illinois, to calibrate and test new assemblies, carbide, and other developmental work on multi-anvil techniques and assemblies.  A portion of the travel budget will also be used to visit other participating multi-anvil laboratories, and to attend the COMPRES annual meeting.  Estimated funds will cover airfare, lodging, transportation as well as costs for per diem.

Examples of Potential Trips:

PI to travel to GSECARS LVP beam line in Argonne, Illinois:

Airfare: $282.95

Transportation: $245.39

Lodging: $275.60

Per Diem: $220.00

TOTAL: $1023.94

The PI is expected to do in average 3 of these trips per year.

PI to travel to attend the COMPRES annual meeting:

Airfare: $250.00

Transportation: $100.00

Lodging: $0

Per Diem: $78.18

TOTAL: $428.18

The PI is expected to do 1 of these trips per year.

Materials and Supplies

We are requesting new molds for pressure media, new ceramics development, and new fabrication development for multi-anvil cell assemblies.  A mold for octahedra with holes and slots is about $3K, while for DIA cubes and other simpler shapes it is less than $1K. Each mold can be used to make about 100,000 pieces (none of our molds have worn out so far).  The molds are stored at Ceramco in New Hampshire where the parts are made. For ceramics development, we will make test extrusions of zirconia for a less expensive zirconia option, especially important for larger cells.  Du-co ceramics is the best option for helping us with this study.  We will also work on extrusions of forsterite as a promising thermal insulator for applications below 14 GPa (20 K per year).

Some of the supplies funding is for carbide testing and development.  We are pursuing promising grades of carbide listed in Ivan Getting’s 1993 paper, some are previously untested and others are used by other laboratories including in Japan.  Targets include grades of Sandvik carbide that are comparable to Toshiba Grade F.  Then there are micro- and nano-grain carbides.  Tomoo Katsura reported at COMPRES this year that hard grades from Fujiloy can be used to greatly extend the pressure as measured by alumina solubility in bridgmanite.  We will try these and equivalent grades from Sandvik that can be found in the Getting paper.  This is a goal of the DELVE initiative of which KL is a member (but that initiative does not include funding for carbide).  Some relatively inexpensive carbides are also available, and one goal will be to see if those have applicability in multi-anvil applications (17.7-19.4 K per year).

Facility User Fees

We have budgeted $6,500 per year for multi-anvil run charges and analytical charges (including x-ray diffraction, SIMS, IR, Raman, SEM, and/or electron microprobe) for analyzing cell assemblies and minerals/materials for this project.

|  | Unit Cost | Usage (no. of hours per year) | Total Cost (per year) |
| --- | --- | --- | --- |
| Multi-anvil Instrument | $150.00 per run | 26 runs | $3900.00 |
| X-ray diffraction Instrument | $16.00 per hour | 50 hours | $800.00 |
| Electron microprobe Instrument | $60.00 per hour | 30 hours | $1800.00 |

Indirect Costs (F&A)

Indirect costs are calculated on Modified Total Direct Costs (MTDC) using F&A rates approved by DHHS. The most current rate is 54.5% for on-campus projects and the rate agreement is dated May 13th, 2016. Items excluded from F&A calculation include: capital equipment, subcontracts over the first $25,000, tuition remission, participant support, rental/maintenance of off-campus space, and patient care fees.

Publication list for 2015 and 2016

Chantel, J.; Manthilake, G.; Andrault, D.; Novella, D.; Yu, T.; Wang, Y.B. (2016)  Experimental evidence supports mantle partial melting in the asthenosphere.  Science Advances 2, UNSP e16000246.

Gaudio, S.J.; Lesher, C.E.; Maekawa, H.; Sen, S. (2015) Linking high-pressure structure and density of albite glasses near the glass transition.  Geochimica et Cosmochimica Acta 157, 28-38.

Guan, L.; Schwartz, M; Zhang, R.; Kroke, E. (2016)  Polymer-precursor-derived (am-) SiC/TiC composites for resistive heaters in large volume multi anvil high pressure/high-temperature apparatus.  High Pressure Research 36, 167-186.

Gullikson, A.; Leinenweber, K.; Stoyanov, E.; Zhang, H.; Malik, A.-S. (2015).  High Pressure Investigation in the System SiO2-GeO2: Mutual Solubility of Si and Ge in Quartz, Coesite, and Rutile-type phases. Journal of the American Ceramic Society 98, 982-989.

Kronbo, C.H.; Nielson, M.B.; Kevy, S.M., Parasiades, P.; Bremholm, M. (2016)  High pressure structure studies of 6H-SrIrO3 and the octahedral tilting in 3C-SrIrO3 towards a post-perovskite.  Journal of Solid State Chemistry 238, 74-82.

Leinenweber, K.; Gullikson, A.; Stoyanov, E.; Malik, A.-S. (2015).  Saturation curve of SiO2 component in rutile-type GeO2: A recoverable high-temperature pressure standard from 3 GPa to 10 GPa.  Journal of Solid State Chemistry 229, 10-18.

Lu, Z.Y.; Li, J. (2015) Melting curve of NaCl to 20 GPa from electrical measurments of capacitive current.  American Mineralogist 100, 1892-1898.

Pommier, A.; Leinenweber, K.; Kohlstedt, D.L.; Qi, C.; Garnero, E.J.; Mackwell, S.J.; Tyburczy, J.A. (2015a)  Experimental constraints on the electrical anisotropy of the lithosphere-asthenosphere system.  Nature 522, 202-206.

Pommier, A.; Leinenweber, K.; Tasaka, M. (2015b) Experimental investigation of the electrical behavior of olivine during partial melting under pressure and application to the lunar mantle.  Earth and Planetary Science Letters 425, 242-255.

Shofners, G.A.; Campbell, A.J.; Danielson, L.R.; Righter, K.; Fischer, R.A.; Wang, Y.B.; POrakapenka, V.  (2016)  The W-WO2 oxygen fugacity buffer (WWO) at high pressure and temperature: Implications for f(O2) buffering and metal-silicate partitioning.  American Mineralogist 101, 211-221.

Wang, S.M.; Antonio, D.; Yu. X.H.; Zhang, J.Z.; Cornelius, A.L.; He, D.W.; Zhao, Y.S. (2015) The hardest superconducting metal nitride.  Scientific Reports 5, article number 13733.

Wang, S.M.; Yu, X.H.; Zhang, J.Z.; Wang, L.P.; Leinenweber, K.; He, D.W.; Zhao, Y.S. (2016)  Synthesis, Hardness, and Electronic Properties of Stoichiometric VN and CrN.  Crystal Growth and Design 16, 351-358.

Wu, Y.; Xu. M.J.; Jin, Z.M.; Fei, Y.W.; Robinson, P.T. (2016)  Experimental constraints on the formation of the Tibetan podiform chromites.  Lithos 245, 109-117.

Zhang, Y.F.; Wang, C.; Wu, Y.; Liu, W.L.; Jin, Z.M. (2015) Experimental constraints on formation of hematite in olivine at high pressures and temperatures.  Physics and Chemistry of Minerals 42, 761-771.

Zhang, Y.; Wu, Y; Wang, C.; Zhu, L.Y.; Jin, Z.M.  (2016) Experimental constraints on the fate of subducted upper continental crust beyond the “depth of no return.”  Geochimica et Cosmochimica Acta 186, 20-225.

Table 1: Examples of Multi-anvil assemblies developed and standardized by the COMPRES Multi-Anvil Development/Cell Assembly Project, along with the numbers of cells provided to the high-pressure research community this year by our project.

| Assembly name | Peak pressure | Peak temperature | Design | # provided 10/2/2015-11/11/2016 |
| --- | --- | --- | --- | --- |
| 6/8 type assemblies |  |  |  |  |
| 8/3 | 25 GPa | 2319 °C | Rhenium furnace | 670 |
| 10/5 | 20 GPa | 2000 °C | Rhenium furnace | 1126 |
| 14/8 “G2” | 13 GPa | 1200 °C | Graphite box furnace | 800 |
| 14/8 step heater | 15 GPa | 1400 °C | Graphite step furnace | 10 |
| 14/8 “HT” | 15 GPa | 2200 °C | Rhenium furnace | 132 |
| 18/12 | 9 GPa | 1500 °C | Graphite box furnace | 395 |
| 8/3 window assembly in-situ | 25 GPa | 2200 °C | LaCrO3 sleeve and rhenium furnace with windows | 0 |
| 10/5 window assembly in-situ | 20 GPa | 2200 °C | LaCrO3 sleeve and rhenium furnace with windows | 55 |
| 10/5 equatorial assembly in-situ | 20 GPa | 1700 °C | TiB2+BN straight furnace, MgO equatorial window, mullite octahedron. | 20 |
| 14/8 “G2” in-situ | 13 GPa | 1200 °C | Graphite box furnace, forsterite sleeve | 20 |
| 14/8 equatorial assembly in-situ | 15 GPa | 1500 °C | TiB2+BN step furnace, MgO equatorial window, mullite octahedron | 65 |
| DIA-type assemblies |  |  |  |  |
| Spherical D-DIA assembly | 6 GPa | 1700 °C | Based on Durham sphere and cradle idea | 75 |
| D-DIA (Deformation DIA) | 8 GPa | 1400 °C | Stony Brook design made at ASU | 50 |
| U-DIA (Ultrasonic DIA) | 8 GPa | 1400 °C | Stony Brook design made at ASU | 0 |
| DIA (regular DIA for diffraction) | 8 GPa | 1400 °C | Stony Brook design made at ASU | 0 |
| ESRF 9 mm assembly | 8 GPa | 1400 °C | Paul Raterron and Nadege Hilairet design | 15 |
| TOTAL |  |  |  | 3483* |

*In addition to items in the Table, 50 14/8 electrical conductivity assemblies were supplied this year.

Table 2: COMPRES Users of cell assemblies in iLab records 2015-2016

| Name | Lab/PI | Institution |
| --- | --- | --- |
| Alwin James | Parise, John | Stony Brook University |
| Anne Pommier | Pommier, Anne | University of California at San Diego |
| Bin Chen | Chen, Bin | University of Hawaii at Manoa |
| Caleb Holyoke | Holyoke, Caleb | University of Akron |
| Dongli Yu | Yu, Dongli | Yanshan University |
| Eun Jeong Kim | Lee, Sung-Keun | Seoul National University |
| Haidong Zhang | Strobel, Timothy | Carnegie Institution for Science |
| Haiyan Chen | Weidner, Donald | Stony Brook University |
| Heather Kirkpatrick | Li, Jie | University of Michigan |
| Hong Yu | Yu, Hong | Starwave Technologies |
| Hui Zhang | Zhang, Hui | Sandvik |
| Huiyang Guo | Gou, Huiyang | HPSTAR |
| Jennifer Kung | Kung, Jennifer | National Cheng Kung University |
| Jeremy Wykes | Wykes, Jeremy | Macquarie University |
| Julien Chantel | Jing, Zhicheng | Case Western Reserve University |
| Kirill Cherednichenko | Solozhenko, Vladimir | Laboratoire des Sciences des Procédés et des Matériaux |
| Kurt Leinenweber | Leinenweber, Kurt | Arizona State University |
| Kyusei Tsuno | Dasgupta, Rajdeep | Rice University |
| Lara Brown | Lesher, Charles | Aarhus University |
| Liping Wang | Wang, Liping | University of Nevada Las Vegas |
| Martin Bremholm | Bremholm, Martin | Aarhus University |
| Matthew Whitaker | Weidner, Donald | Stony Brook University |
| Mei Wang | Wang, Mei | TJ Pegasus |
| Pamela Burnley | Burnley, Pamela | University of Nevada Las Vegas |
| Paul Raterron | Raterron, Paul | Brown University |
| Robert P. Rapp | Rapp, Robert P. | Australian National University |
| Samantha Clarke | Freedman, Danna | Northwestern University |
| Tony Yu | Wang, Yanbin | University of Chicago |
| William Durham | Durham, William | Massachusetts Institute of Techn |
| William Li | Li, Pan | PES Enterprise Inc. |
| Xintong Qi | Li, Baosheng | Stony Brook University |
| Xiancheng Wang | Wang, Xiancheng | Institute of Physics, The Chinese Academy of Sciences |
| Zhenmin Jing | Jing, Zhenmin | China University of Geosciences (Wuhan) |
|  |  | Jilin University |
|  |  | Jilin University Superhard Materials |
|  |  | Zhengzhou University |