IXS at Extreme Pressure and Temperature at 3-ID of the APS

2016 COMPRES Annual Report

November 2015 – October 2016

Prepared by:  Ercan Alp (ANL) and Jay Bass (UIUC) (PI)

Wenli Bi (COMPRES researcher)

Michael Hu, Jiyong Zhao, Ahmet Alatas, Thomas Toellner, Ayman Said (ANL-APS staff)

Overview

The inelastic x-ray scattering (IXS) at extreme temperature and pressure (between 10 and 3500 K) is a unique capability provided to COMPRES Users through by General User Proposals as well as a COMPRES Partner User Proposal at the APS 3-ID beamlines.

The capability involves access to two types of instruments, i) nuclear resonant scattering at 3-ID-B, 3-ID-D, and 30-ID-C ii) HERIX-3 and HERIX-30: High Energy Resolution Inelastic X-Ray Spectrometer at 3-ID-C, 30-ID-C.

The special capabilities involve

- micro-focused beam with 10 microns beamsize at 3-ID-B and 17 microns beam size at 3-ID-C, and 30 microns at 30-ID-C,

- 1 meV or better resolution monochromators for 57Fe, 119Sn, 151Eu, 161Dy, 83Kr Mössbauer isotopes, and a cryogenically cooled high-resolution monochromator at 21.65 keV for the HERIX-3 and HERIXS-30 instruments,

- on-line Ruby pressure calibration system in 3-ID-B and 30-ID-C and offline Raman system,

- symmetric DACs for synchrotron Mössbauer experiment and screw- and membrane-type panoramic DAC’s for nuclear resonant inelastic x-ray scattering experiments,

- cryogenically cooled cryostat suitable for cooling samples under pressure for magnetic studies,

- combined inelastic spectrometer/x-ray diffraction capability for single-crystal structure and orientation determination inside the DAC at HERIX-3 and HERIX-30 spectrometers,

- combined nuclear resonant inelastic spectrometer with Mössbauer spectroscopy and x-ray diffraction capability for atomic and electronic structure determination, and equation of state determination,

- high temperature, double-sided, fiber IR- laser heating capability to reach above 3500 K, and internal resistive heating capability to reach 1000 K. Includes spectroradiometric temperature measurements determined from CCD and FasTeR spectrometers.

In addition, we offer a conventional Mössbauer spectroscopy (CMS) laboratory with two Mössbauer drives as another resource, one that is used frequently by COMPRES researchers. With the availability of point-sources, spectra can be taken from high pressure samples inside a DAC or recovered samples after high pressure experiments.

Scientific Highlights

Velocity-density relation of hcp-Fe alloyed with Ni and Si in the Earth inner core: Iron alloyed with Ni and Si has been suggested to be a major component of the Earth’s inner core. High-pressure results on the combined alloying effects of Ni and Si on seismic parameters of iron are essential for establishing satisfactory geophysical and geochemical models of the region. Combining HERIX and NRIXS experiments, the compressional (Vp) and shear wave velocity-density relations have been investigated in hcp-Fe and hcp-Fe86.8Ni8.6Si4.6 alloy up to 206 GPa and 136 GPa, respectively. The results show that the combined addition of Ni and Si slightly increases the Vp but significantly decreases the Vs of hcp-Fe at a given density relevant to the inner core. Such distinct alloying effects on velocities of hcp-Fe produce a high Poisson’s ratio (ν) for the alloy at inner core densities. Analysis of the available high P-T results on Vp and Vs of Fe alloy with light elements shows that high temperature can further enhance the ν of hcp-Fe alloyed with Ni and Si. Most significantly, the derived seismic heterogeneity ratios of this hcp alloy present a better match with global seismic observations. The results provide a multifactorial geophysical constraint on the compositional model of the inner core which is consistent with silicon being a major light element alloyed with Fe and 5 wt % Ni (Liu et al., 2016).

Temperature of Earth’s core constrained from melting of Fe and Fe-Ni: The melting temperature of pure iron and iron alloyed with a few percent nickel provide the basis for understanding expected temperatures in the core. The temperature of the core at the core-mantle boundary (CMB) provides critical constraints on the heat flow across the boundary and thus affects the age determination of the inner core, influences the evolution of the geodynamics, and controls the temperature gradient within the thermal boundary layer above the CMB. However, high pressure melting curve of fcc-Fe and hcp-Fe are still controversial, leading to large variation of estimated temperature at the inner core-outer core boundary. Synchrotron Mossbauer spectroscopy has been applied to study melting and provide a unique diagnostic tool for solid-liquid transition of Fe-rich materials. High pressure melting curves of fcc- and hcp-structured Fe0.9Ni0.1 and Fe up to pressures of 125 GPa has been constructed. The upper bound for the temperature on the core side of the CMB was derived to be 4000±200 K. With the addition of light elements such as a combination of sulfur, oxygen, and silicon, the core-side CMB temperature could be as low as 3500 K. These newly constrained temperature bounds will help determine the range of permissible phase assemblages and transport at the core–mantle and inner core–outer core boundaries (Zhang et al., 2016).

Spin crossover of (Mg,Fe)O at high pressure: Iron-bearing periclase is thought to represent a significant fraction of Earth’s lower mantle. However, the concentration of iron in (Mg,Fe)O is not well constrained at all mantle depths. An understanding of the effect of iron on the density, crystal structure, and magnetic behavior of ferropericlase is important for understanding the presence of (Mg,Fe)O in lower mantle phase assemblage. X-ray diffraction and synchrotron Mössbauer spectroscopy techniques have been applied to obtain the equation of state to 83 GPa and hyperfine parameters to 107 GPa at 300 K. The study found a gradual volume drop corresponding to a high-spin to low-spin crossover between 45 and 83 GPa. The results imply that the iron-rich (Mg,Fe)O at the core-mantle boundary would likely contain a significant fraction of high-spin (less dense) iron, contributing a positive buoyancy to promote observable topographic relief in tomographic images of the lowermost mantle (Solomatova et al., 2016).

Spinel–olivine–pyroxene equilibrium iron isotopic fractionation: Partitioning of ferrous and ferric iron between different minerals and melts can control redox conditions in igneous rocks. The Mg and Fe isotope composition of spinels is is emerging as a new petrogenetic tracer. NRIXS as well as conventional Mössbauer spectroscopy experiments show a single redox-dependent calibration line accounts for the effect of Fe3+/Fetot on the reduced isotopic partition function ratios of the spinels, which successfully describes the equilibrium Fe isotopes fractionation factors between spinels and silicates (olivine and pyroxenes). The calibration applies to the entire range of Fe3+/Fetot ratios found in natural spinels and provides a basis for interpreting iron isotopic variations documented in mantle peridotites. Except for a few exceptions, most of the samples measured so far are in isotopic disequilibrium, reflecting metasomatism and partial melting processes (Roskosz et al., 2015).

Beamline Personnel

The 3-ID beamline is owned and operated by the X-ray Science Division of the Advanced Photon Source. There are 5 full-time staff paid by the APS, and a Visiting Spectroscopist researcher (Wenli Bi), jointly funded by COMPRES (70%) and the APS (30%). In addition, the beamline has full access to APS’s technical and engineering support system. The group is part of Inelastic X-Ray scattering group, led by Dr. Thomas Gog, who reports to Dr. Jonathan Lang, the Director of XSD at the APS.

Beamline Operations

Experiments are performed at the 3-ID and 30-ID beamlines based on general user proposals (GUP’s). The oversubscription rate at 3-ID is 2.5.  During the period of this report, there were 27 and 15 GUP requests from Earth science and high-pressure research groups for 3-ID and 30-ID, respectively. Of these GUP requests 19 (70 % of the total requests) and 7 (47 %) were granted beam time at Sector 3-ID and 30-ID, respectively. These percentages are well above the average acceptance rate, which is below 30%. The relatively high success rates for COMPRES proposals are due to the excellent quality of research that these techniques attract from the COMPRES community, and partly due to the close collaboration between the beamline staff and user groups to develop effective proposals that are highly competitive for beam time. During the past two years, 14 COMPRES user groups have been allocated beam time. About 25 graduate and 4 undergraduate students participated experiments.

In addition, beam time is also available through partner user proposal. The PUP partnership with COMPRES titled “Development of High-Pressure and Low-Temperature Nuclear Resonant Scattering Capability for Geoscience and Condensed Matter Physics Applications at 3-ID” concluded in Feb. 2016. The PUP carried a weight of 10 %, or about 1 week in each cycle, with a total of 3 weeks each year for two years. From this PUP we have developed the low temperature and high-pressure NRIXS capability at sector 3. During the past year, 4 user groups have utilized this system for low temperature and high-pressure studies.

The offline Mössbauer lab provides mail-in service for COMRPES users for beamline related experiments as well as individual sample characterization. It has been in high demand. In the last year over 30 samples from 18 user groups have been measured.

There has been a significant increase in the beam time allocated for nuclear resonant scattering. The share of NRS at 3-ID has gone up from 60 to 80 % with the inclusion of hybrid mode experiments. Furthermore, 10 % additional beam time is now allocated for 119Sn experiments at Sector 30, which reduces the burden on Sector 3, and allows more time COMPRES users. Finally, the total time for IXS measurements between sector 3 and 30 has gone up from 70 % to 90 %. These increases are due to relocation of the MERIX instruments at Sector 30 to Sector 27, a new sector. This sector, which is dedicated to electronic RIXS measurements, also has high-pressure capabilities, which is a potential expansion point.

Performance Metrics

In the period of this report 13 groups from geosciences/COMPRES community were allocated beam time at 3ID and 30ID. The average time allocated for COMPRES users is ~40% of the total available beam time. Detailed information of the allocation is listed in the Appendix.

Beamline Community Activities

- Workshop on Nuclear Resonant Scattering and Data Analysis, Nov. 11-13, 2016 at the Advanced Photon Source, Argonne National Laboratory. The workshop is funded by COMPRES and APS. 37 participants registered for the workshop.

- Active participation in the COMPRES annual meeting.

- Year around training of users for data analysis.

- Over 30 samples from COMPRES users have been measured and characterized in the Mössbauer lab.

- Beamline staff is heavily involved with beam time proposal writing, diamond anvil cell preparation, sample loading, data collection, analysis, and data interpretation and manuscript writing for users.

Beamline Development

- Development of a cryostat and a miniaturized gas membrane-driven DAC for low temperature high-pressure NRXIS and SMS experiments, the first and only instrument of its kind in the world.

- Development and establishing the Sn Mössbauer isotope studies under high-pressure conditions at Sector 30. This is the strongest nuclear resonant source for Sn worldwide by a factor of ten.

- Exploitation of the APS hybrid mode with extended time window to achieve SMS data with improved accuracy. The case has been established from studies of α-ε transition in iron from 10 K to 600 K.

- Re-commissioning of 2 meV resolution high resolution monochromator for iron to facilitate force constant and isotope fractionation experiments under high-pressure

- An offline Raman system that was previously located at GSECARS and recently moved to sector 3. This system allows users to measure pressure from ruby fluorescence and diamond Raman edge at extreme pressures (~1Mbar and above) as well as taking Raman spectra from samples under pressure. The latter provides complimentary information to the NRIXS and IXS data about pressure-induced phase transitions.

Planned Activities

- _Upgrade of the high-heat-load monochromator ($200k, paid by ANL-LDRD)

- Fast chopper system for Synchrotron Mössbauer Spectroscopy ($500k, paid by ANL)

- Acquire a new 119Sn source (LDRD funds from ANL)

- Acquire a modern Mössbauer spectrometer (LDRD funds from ANL)

Budget

We request partial support (at the 2/3 level) for a scientist (Dr. Wenli Bi) to work at Sectors 3, 30 and the offline Mössbauer lab. Dr. Bi will be: 1) developing new high pressure capabilities at Sector 3 and 30 beamlines and Mössbauer lab; 2) working with the COMPRES community in developing competitive proposals for beam time; 3) assisting COMPRES users of Sector 3 and 30 during their beam time; 4) performing on-line analysis of their results (which is often formidable for even experienced users); 5) education and outreach through organizing workshops; 6) assisting users of the conventional Mössbauer spectrometer; 7) writing instruction manuals for the complex software used in analyzing results; 8) maintaining the offline Raman system. Dr. Bi will be a primary contact for the COMPRES community with Sectors 3, 30 and the offline CMS lab.

|  | - 2016-2017 |  | - 2017-2018 |
| --- | --- | --- | --- |
| - Wenli Bi |  | - 46,007 | - 47,387 |
| 8 months |  |  |  |
| - Fringe Ben. |  | - 20,452 | - 21,065 |
| - Sal.+Fringe |  | - 66,459 | - 68,452 |
| - Travel, Dom. |  | - 6,750 | - 7,000 |
| - Mat. & Supp. |  | - 2,552 | - 2,613 |
| - Conf. Regist. |  | - 1,050 | - 1,050 |
| - Tot. Dir. Cost |  | - 76,811 | - 79,115 |
| - IDC |  | - 19,971 | - 20,570 |
| Total |  | - 96,782 | - 99,685 |

Appendices

- List of publications

- Roskosz, M., Sio, C. K. I., Dauphas, N., Bi, W., Tissot, F. L. H., Hu, M. Y., Zhao, J., Alp, E. E. (2015). Spinel–olivine–pyroxene equilibrium iron isotopic fractionation and applications to natural peridotites. Geochim. Cosmochim. Acta 169, 184.

- Wolf, A. S., Jackson, J. M., Dera, P., Prakapenka, V. B. (2015). The thermal equation of state of (Mg, Fe)SiO3  bridgmanite (perovskite) and implications for lower mantle structures. J. Geophys. Res. 120 (11), 7460.

- Wicks, J.K., Jackson, J. M., Sturhahn, W., K.K Zhuravlev, Tkachev, S. N., and Prakapenka, V. B. (2015). Thermal equation of state and stability of (Mg0.06Fe0.94)O. Phys. Earth and Planet. Interiors, 249, 28.

- Liu, J. (2015). The role of iron in the Earth's deep interior. Ph.D. -Thesis, University of Texas at Austin.

- Mauger, L. M. (2015). Phonon thermodynamics of iron and cementite. Ph.D.-Thesis, California Institute of Technology.

- Liu, H., Tse, J. S., Hu, M. Y., Bi, W., Zhao, J., Alp, E. E., Pasternak, M., Taylor, R. D., Lashley, J. C. (2015). Mechanisms for pressure-induced crystal-crystal transition, amorphization, and devitrification of SnI4. The Journal of Chemical Physics 143 (16), 164508.

- Liu, J., Lin, J-F., Alatas, A., M. Hu, Zhao, J., Dubrovinsky, L. (2016). Seismic parameters of hcp-Fe alloyed with Ni and Si in the Earth’s inner core. J. Geophys. Res. Solid Earth, 121, 610 (2016).

- Solomatova, N. V., Jackson, J. M., Sturhahn, W., Wicks, J. K., Zhao, J., Toellner, T. S., Kalkan, B., Steinhardt, W. M. (2016). Equation of state and spin crossover of (Mg,Fe)O at high pressures, Am. Mineral. 101, 1084.

- Bi, W., Li, Fabbris, J. M., G., Zhao, J., Haskel, D., Alp, E. E., Hu, M. Y., Chow, P., Xiao, Y., Xu, W., Schilling, J. S. (2016). Magnetism of europium under extreme pressures. Phys. Rev. B  93, 184424.

- Zhang, D., Jackson, J. M., Zhao, J., Sturhahn, W., Alp, E. E., Hu, M. Y., Toellner, T. S., Murphy, C. A., Prakapenka, V. B. (2016). Temperature of Earth's core constrained from melting of Fe and Fe0.9Ni0.1 at high pressures. Earth Planet. Sci. Lett.  447, 72.

- Wang, Y. Q., Lu, P. C., J. J. Wu, Liu, J., Wang, X. C., Zhao, J. Y., Bi, W., Alp, E. E., Park, C. Y., Popov, D., Jin, C. Q., Sun, J., Lin, J.-F. (2016). Phonon density of states of single-crystal SrFe2As2 across the collapsed phase transition at high pressure. Phys. Rev. B 94, 014516.

- Hu, M. Y., (2016). Some notes on data analysis for nuclear resonant inelastic x-ray scattering. Hyperfine Interact.  237, 64.

- Li, C., Yu, Z., Bi, W., Zhao, J., Hu, M. Y., Zhao, J., Wu, W., Luo, J., Yan, H., Alp, E. E. & Liu, H. (2016). Phys. B Condens. Matter. 501, 101–105.

- Chen, B.,  Li, J. (2016). Deep Earth: Physics and Chemistry of the Lower Mantle and Core, H. Terasaki, R.A. Fischer, eds., Wiley, 2016, 277 - 288.

- Chen, B., Lin, JF. Chen, J., Zhang, H., Zeng, Q., (2016) Synchrotron-based high-pressure research in materials science MRS Bull.  41, 473-478.

- List of proposals granted beamtime and usage

Please find the attached excel file titled ”3ID_Users_Statistics”.

- The beamtime schedule for the last year’s cycles

Please refer to the attached excel file titled ”3ID_beam_schedule_2016”.

- List of users in the Mössbauer lab from geosciences and high pressure physics

Community.

| PI | Institution | PI | Institution |
| --- | --- | --- | --- |
| Thomas Duffy | Princeton Univ. | Eran Greenberg | GSECARS |
| John Tse | Univ. of Saskatchewan | Dan Shim | Arizona State Univ. |
| Jay Bass | Univ. of Illinois Urbana-Champaign | Ercan Alp | Argonne National Lab |
| Shun-ichiro Karato | Yale Univ. | Barbara Lavina | UNLV |
| TakamitsuYamanaka | CIW | Kanani Lee | Yale Univ. |
| Jennifer Jackson | Caltech | Michael Krawczynski | WashU |
| Susannah Dorfman | Michigan State Univ. | Nicholas Dauphas | Univ. of Chicago |
| Jung-Fu Lin | Univ. of Texas at Austin | Mathieu Roskosz | Muséum National d'Histoire Naturelle Mineralogy |
| Christy Till | Arizona State Univ. | Jackie Li | Univ. of Michigan |