Diamond-Anvil Cell Infrared Facility at the National Synchrotron Light Source II

2015 COMPRES Annual Report

November 2014 – October 2015

Prepared by Zhenxian Liu and Russell J. Hemley

Geophysical Laboratory, Carnegie Institution of Washington

Overview

The diamond anvil cell (DAC) infrared (IR) beamline at the National Synchrotron Light Source (NSLS-U2A), developed/managed by Geophysical Laboratory, Carnegie Institution of Washington through the NSF-COMPRES support, was an integrated and dedicated facility with the capacity for synchrotron infrared micro-spectroscopic techniques, Raman scattering, and visible spectroscopy combined with diamond-anvil cell methods for the measurement of far- to near-IR and visible spectra of materials from ambient to ultrahigh pressures at variable temperatures. The NSLS-U2A facility was the first dedicated high-pressure IR beamline in the world, and it is a COMPRES showcase for the worldwide earth science and high-pressure research communities. The presence of an IR beamline together with x-ray facilities for high-pressure experiments was one of the unique features of the NSLS for general users. We provided convenient access for users from the COMPRES community and to further broaden our user base. In addition, we promote our users’ research projects on problems relating to high-pressure geoscience and planetary science, complemented by studies in materials science, condensed-matter physics, and chemistry (many of which are also carried out by the COMPRES user community). The major beamline upgrades completed during 2009-2011 provided by the support of COMPRES and the Carnegie-DOE Alliance Center (CDAC) have not only significantly improved the beamline performance but also made it available for users to address problems ranging from outer solar system bodies to the Earth’s core.

As NSLS ceased its operation on September 30, 2014 and NSLS-II achieved “first light” on October 23, 2014, the high-pressure infrared facility/programs are currently moving forward from NSLS to NSLS-II with a brighter and more stable synchrotron IR source. The Frontier Synchrotron Infrared Spectroscopy Beamline under Extreme Conditions (FIS), as one of the five BDN (beamlines developed by NSLS-II) beamlines, is the successor of NSLS-U2A currently under development at NSLS II. To bridge the gap between NSLS and NSLS-II, an offline infrared lab including FTIR and Raman spectroscopic systems have been established and are available for users during the transition period. Moreover, an Advanced Program proposal for developing high-pressure IR programs at Advanced Light Source has been approved and a custom IR microscope system has been set up. Up to 50% of the AP beamtime will be allocated to COMPRES users as needed.

Scientific Highlights

a. New Carbon Bonding under Extreme Conditions

Only a small fraction of our planet’s total carbon is found at the surface. In fact, Earth’s mantle is thought to be the largest carbon reservoir. Carbonates, and in particular ferromagnesite ((Mg,Fe)CO3), are likely candidates for deep-Earth carbon storage and therefore play a key role in the deep carbon cycle. The behavior of these carbonates at the high pressure and temperature conditions of Earth’s interior is therefore of great interest for understanding global carbon cycling and storage.

A new study led by E. Boulard (Stanford University) reports unequivocal experimental evidence for tetrahedrally coordinated carbon in high pressure carbonates, obtained by combined experimental studies at U2A beamline, National Synchrotron Light Source, Brookhaven National Laboratory and theoretical calculations at University of Chicago. With the combination of in situ synchrotron infrared spectroscopic measurements carried out at high pressure, and ab initio calculations on the ferromagnesite structure, it was possible to identify a unique vibrational signature present only in the high-pressure phase, and thus a new carbon-oxygen bond forms in ferromagnesite under high pressure and temperature. The new vibrational signature is assigned to tetrahedrally coordinated carbon atoms with asymmetric C-O bonds.

Ferromagnesite represents an important rock-forming mineral, fundamentally distinct from silicates in Earth’s crust. At low pressure, carbon bonds to three oxygen atoms, while silicon bonds to four. Tetrahedrally coordinated carbonates likely exhibit dramatically different behavior compared to three-fold coordinated carbonates, including altered reactivity with other mantle phases and changes in chemical melt properties. This bonding behavior could therefore have significant implications for carbon reservoirs and fluxes, as well as for our understanding of the global geodynamic carbon cycle.

Figure 1. In-situ synchrotron infrared measurements at high pressure. (a) Experimental infrared spectra collected on compression of the ferromagnesite (black lines) at 0 and 54 GPa, and on decompression of the post-magnesite phase (red lines) at 58 and 0 GPa. The region between 1,900 and 2,300 cm-1 is dominated by absorption from the diamond anvils. The inserts show carbon environment and the vibrational mode ~1,252 cm-1 (marked by the arrow) identified as a unique signature of the high-pressure phase. (b) Experimental infrared spectra collected on the decompression of the post-magnesite phase from103 to 0 GPa. The scale bars give the absorbance scale for each panel.

Reference:  E. Boulard, D. Pan, G. Galli, Z. Liu and W. Mao, Tetrahedrally Coordinated Carbonates in Earth’s Lower Mantle, Nat. Commun., 6, 6311 (2015).

# b. Tracing O-H Stretching Behavior in Amphiboles by Synchrotron Infrared Spectroscopy under High Pressure

Oceanic plates contain numerous hydrous phases including amphiboles, which are important carriers of water into subduction zones. The hydroxyl bound within the crystalline structure of hydrous minerals, as well as changes in hydrogen bond symmetry, can significantly impact the bulk properties of these minerals. For example, a dehydration reaction is initiated when amphiboles within the oceanic slab are subjected to increasing temperatures and pressures as the slab subducts along the cold slab geotherm. Hydrogen cycling in the Earth’s interior is critical due to the influence of hydroxyl on melting temperatures, rheology, electrical conductivity, and atomic diffusivity, as well as macro-scale phenomena including plate tectonics and volcanism.

Figure 2. (a): Comparison of spectral peak widths using a non-hydrostatic KBr pressure medium versus a quasi-hydrostatic Ne pressure medium. Both sets of measurements were made using the same mineral specimen (tremolite). Sample L13 (blue markers) was loaded in Ne and sample L14 (red markers) was loaded using a KBr pressure medium. The same colors are used to identify samples within inset. (b): Comparison of pressure dependence of O-H stretching frequency in amphiboles and t–o–t (talc and mica-like) phyllosilicates. Closed circles: this study; different shades of infill indicate the A, B, C, and D bands. Open circles: other amphiboles, talc, and micas. All of these minerals containing analogous hydrogen bonding sites, at the non-tetrahedral oxygen in t–o–t layers. Error bars represent 1σ uncertainties.

Combined synchrotron IR and DAC techniques, a new study led by Lily Thompson (University of Chicago) examines twelve natural amphibole samples spanning a range of ten compositions at room temperature and pressures up to 60 GPa. Infrared spectra were collected at atmospheric pressure and at regular intervals during compression, allowing for the collection of spectra centered on the typical O-H stretching region at 3600-3700 cm-1 as they evolved with pressure for each composition. Interestingly, the number of O-H bands within each sample was found to vary with composition, but the pressure dependence of O-H frequency shifting more closely correlated with mode frequency at ambient pressure than with composition. Combined with earlier results, these data reveal a linear relationship between mode frequency at ambient pressure and pressure dependence of O-H stretching modes in amphiboles and sheet silicates.

Two sample preparation methods utilized in this study allowed for direct comparison between quasi-hydrostatic neon-loaded sample conditions and the conditions achieved with a KBr pressure medium. Samples loaded in neon preserved sharper peaks, allowing greater spectral resolution, especially at higher pressures when peaks are most likely to broaden or disappear due to crystalline disorder and pressure gradients across the sample. This new quasihydrostatic loading method proved valuable to tracing O-H stretching behavior in amphiboles to higher pressures than previously obtained and will lend itself to future study of O-H stretching pressure dependence in a wide range of hydrous minerals.

Reference: E.C. Thompson, A.J. Campbell and Z. Liu, In-situ infrared spectroscopic studies of hydroxyl in amphiboles at high pressure, Am. Mineral., accepted.

c. Dry (Mg,Fe)SiO3 Perovskite in the Earth’s Lower Mantle

The [OH−] water content of the lower mantle is uncertain, yet it has far-reaching impacts on the physical behavior of the mantle and our planet’s evolution. Small amounts of dissolved water in the major nominally anhydrous silicates of the upper mantle, including the transition zone, are shown to have significant effects on the mantle’s deformation, elastic moduli, and relaxation. Further, the incorporation of water in olivine polymorphs increases the reaction kinetics of phase transformations with implications for deep-focus earthquakes and mass transfer across the base of the transition zone. The transition zone has a large water storage capacity; that is, it can incorporate 1~2 wt% H2O before inducing melting, in which the major carriers of water in the transition zone are the olivine polymorphs, wadsleyite, and ringwoodite. Recent evidence from deep diamond suggests that at least portions of the transition zone contain up to 1 wt % H2O.

(a)

(b)

(c)

Figure 3. (a) Zero-pressure IR spectra of perovskite synthesized from 3941 as a function of synthesis pressure. For comparison, the lower curve is the IR spectrum of 1009Da (14,516 starting sample) vertically expanded by a factor of 59 (the ratio of OH content of the two starting samples). The sharp peaks (*) are due to C–H on the optical system. (b) Decompression of sample 0608Ha laser heated at 42 GPa in an argon pressure medium to ambient pressure. No major changes in features are observed, with a negative shift of the main OH band with a slope of ~4.2 cm-1/GPa. Additional bands in the O–HO bending region have a positive shift with the 2216 cm-1 band with a +12.3 cm-1/GPa, 1444 cm-1 and 1564 cm-1 peaks with relatively weak shifts of +2.4 cm-1/GPa, and +2.6 cm-1, respectively. (c) IR spectra of sample 0608Ha cooled to 12 K at 10-6 torr after synthesis and recovery. Four peaks become evident below 100 K, and three peaks are retained upon returning to room temperature.

For the lower mantle, however, hydration of the ABO3 rare earth perovskites (A2+ and B4+ cations) shows that significant hydration with as much as 2 wt % H2O can occur with B-site aliovalent substitutions at temperatures above 1300 K. These proton conducting, acceptor-doped perovskites accommodate the aliovalent cation via oxygen vacancies, such that many of these solid oxide fuel cell materials are significantly oxygen deficient. Combined synthesis experiments and first-principles calculations, a team led by W. Penaro (Ohio State University) found that MgSiO3-perovskite with minor Al or Fe does not incorporate significant OH under lower mantle conditions. Perovskite, stishovite, and residual melt were synthesized from natural Bamble enstatite samples (Mg/(Fe + Mg) = 0.89 and 0.93; Al2O3<0.1 wt % with 35 and 2065 ppm weight H2O, respectively) in the laser-heated diamond anvil cell at 1600~2000 K and 25~65 GPa. Combined synchrotron IR spectroscopy, X-ray diffraction, and ex situ transmission electron microscopy analysis demonstrates little difference in the resulting perovskite as a function of initial water content. Four distinct OH vibrational stretching bands are evident upon cooling below 100 K (3576, 3378, 3274, and 3078 cm-1), suggesting four potential bonding sites for OH in perovskite with a maximum water content of 220 ppm weight H2O, and likely no more than 10 ppm weight H2O. Complementary, Fe-free, first-principles calculations predict multiple potential bonding sites for hydrogen in perovskite, each with significant solution enthalpy (0.2 eV/defect). The calculations also reveal that perovskite can dissolve less than 37 ppm weight H2O (400 ppm H/Si) at the top of the lower mantle, decreasing to 31 ppm weight H2O (340 ppm H/Si) at 125 GPa and 3000 K in the absence of a melt or fluid phase. These results resolve a long-standing debate of the perovskite melting curve and explain the order-of-magnitude increase in viscosity from upper to lower mantle.

Reference: W. Panero, J. Pigott, D. Reaman, J. Kabbes, and Z. Liu, Dry (Mg,Fe)SiO3 Perovskite in the Earth's Lower Mantle, J. Geophys. Res., 120(3), 894–908 (2015).

# d. Pressure Induced Metallization without Structural Transition in Layered MoSe2

The electronic structure transition (insulator to metal or semiconductor to metal transition) is generally accompanied or followed by a first-order structural transition. However, using multiple experimental techniques including synchrotron IR spectroscopy and ab-initio calculations, a team of researchers led by Z. Zhao (Stanford University) discovered that the metallization process does not involve any crystal structure change in MoSe2, which allows its energy continuous tunability for potential opto-electronic or photovoltaic applications. Transition metal dichalcogenides (TMDs) have attracted intense scientific and engineering interest since the discovering of graphene in 2004. In the past decade, people mainly use three experimental methods: applying electrical field, utilizing quantum confinement with samples thinning down into monolayers, or employing stress or strain to vary the electronic structure of TMDs. In recent years, pressure is found to be a powerful tool for tuning mechanical and optical properties in TMDs, eg. MoS2.

At ambient conditions, MoS2 and MoSe2 are isostructural in crystal structures and have highly similar electronic structures, so it’s nature to assume a similar structure transition would also occur in MoSe2 just like that happened in MoS2. However, in this new study, both high-pressure x-ray diffraction data and Raman spectroscopy indicated no crystal structure change on MoSe2 up to ~60 GPa. To explore the observed stability of MoSe2, the team carry out ab-initio simulation, find that the energy barrier for layer sliding in MoSe2 is much higher than that in MoS2, and the high-pressure phase of MoSe2 bear higher energies than the initial structures, which possibly make the crystal-structure transition unfavorable. Additionally, somewhat more surprising is the metallization of MoSe2 at about 40 GPa from high-pressure IR spectroscopy and temperature-dependent resistivity measurements. These results demonstrate that pressure doesn’t result in crystal structure change but the electronic structure transition, which is rarely seen in TMDs. Through high-pressure IR spectroscopy, electrical resistivity measurement combined with ab-initio calculations, the team find that the band-gap of MoSe2 (in the range of visible to IR region) as well as its electronic structure both exhibit strong dependence on pressure.

Figure 4. IR transmittance spectra and analysis. (a) Representative IR transmittance spectra at high pressures, numbers show pressures in unit of GPa. The 0.23–0.28 eV region is obscured by diamond phonon absorptions from the anvils. (b) Pressure-energy-optical density (OD) contour, OD is defined as -log(T) while T is the transmittance. (c) Evolution of band gap under pressure. Circles are extrapolated indirect band gaps and the curve shows a parabolic fit of the band gap versus pressure. The band gap closure is observed at ~40 GPa.

Reference:  Z. Zhao, H. Zhang, H. Yuan, S. Wang, Y. Lin, Q. Zeng, G. Xu, Z. Liu, G. Solanki, K. Patel, Y. Cui, H. Hwany, and W. Mao, Pressure Induced Metallization with Absence of Structural Transition in Layered Molybdenum Diselenide, Nat. Commun., 6, 7312 (2015).

e. High Pressure-Temperature Phase Diagram of 1,1-Diamino-2,2-dinitroethylene (FOX-7)

1,1-Diamino-2,2-dinitroethylene (C2H4N4O4), commonly referred to as FOX-7 or DADNE, is among the less sensitive high explosives. Though more sensitive than 1,3,5-triamino-2,4,6-trinitrobenzene (TATB), it has performance comparable to that of related secondary high explosives hexahydro-1,3,5-trinitro- 1,3,5-triazine (RDX) and octahydro-1,3,5,7-tetranitro-1,3,5,7-tetrazocine (HMX). To better understand the mechanisms behind high explosive (HE) performance and sensitivity, it is important to explore the structural phase stability across a broad range of pressure (P) and temperature (T) conditions. Due to the high cost and complexity of performing experiments on HE, computational models are relied upon to help expand our understanding of the mechanisms behind performance, sensitivity, and safety. However, accurate models of the complex behavior of HEs require experiments to be performed from ambient P−T up to detonation conditions. FOX-7 is a relatively small and simple molecule, making it an ideal energetic material to investigate both experimentally and theoretically.

Using in situ synchrotron IR spectroscopy and resistively heated DAC, a team led by T. Bishop (University of Alabama at Birmingham) and N. Velisavljevic (Los Alamos National Laboratory) extensively studied the pressure−temperature (P−T) phase diagram of FOX-7 up to 360 °C and 10 GPa. The stability of high P−T FOX-7 polymorphs is established from ambient pressure up to 10 GPa and temperatures until decomposition. The phase diagram indicates two near isobaric phase boundaries at ∼ 2 GPa (α→I) and ∼ 5 GPa (I→II) that persists from 25 °C until the onset of decomposition at ∼ 300 ° C. In addition, the ambient pressure, high-temperature α → β phase transition (∼ 111 °C) lies along a steep boundary (∼ 100 ° C/GPa) with a α−β−δ triple point at ∼ 1 GPa and 300 ° C. A 0.9 GPa isobaric temperature ramping measurement indicates a limited stability range for the γ -phase between 0.5 and 0.9 GPa and 180 and 260 °C, terminating in a β−γ−δ triple point. With increasing pressure, the δ-phase exhibited a small negative dT/dP slope (up to ∼ 0.2 GPa) before turning over to a positive 70 °C/GPa slope, at higher pressures. The decomposition boundary (∼ 55 °C/GPa) was identified through the emergence of spectroscopic signatures of the characteristic decomposition products as well as trapped inclusions within the solid KBr pressure media.

(a)

(b)

Figure 5. (a): Mid-IR spectra taken at incremental temperatures near 2.0 GPa isobar. The pressure and temperature of each spectrum are indicated on the right side of the figure with the sample decomposition indicated in red. (b) Mid-IR spectra taken at incremental temperatures along a 300 °C isotherm. The pressure and temperature of each spectrum are indicated on the right side of the figure. Approximate structural phase transitions are indicted with a change in color of the spectra at the respective pressure and temperature: α-phase (purple), β-phase (blue), phase-I (black), and phase-II (wine).

Figure 6. P−T phase diagram of FOX-7 based on collected infrared data presented in this work and available literature values. The solid lines indicate approximate phase boundaries for the indicated phase(s), whereas dashed lines are approximate extensions of these boundaries based on observed vibrational behavior and/or the development of characteristic decomposition gases. Colored symbols represent the spectrum where the characteristic vibrational behavior of the indicated phase is first observed, and the error in the observation is considered to be the step rate of the experiment (gray symbol just prior to the colored symbol - along the experimental path) unless otherwise indicated. The color scheme is as follows: β-phase (blue), γ-phase (dark-yellow), δ- phase (purple), phase-I (green), phase-II (wine), and decomposition (red). The phase diagram suggest two potential triple points at high P-T: β +γ +δ and α +β +δ.

Reference: M. Bishop, N. Velisavljevic, R. Chellappa, Y. Vohra, High Pressure−Temperature Phase Diagram of 1,1-Diamino-2,2- dinitroethylene (FOX-7), J. Phys. Chem. A, 119(37), 9739−9747 (2015).

Beamline Personnel

The management team from the Carnegie Institution of Washington is headed by PIs Russell J. Hemley (Senior Staff Member, Geophysical Laboratory) and Zhenxian Liu (Research Scientist, Geophysical Laboratory). Carnegie also serves as the funding host for the project.

Beamline Scientist

Zhenxian Liu was appointed as Research Scientist/Beamline Scientist at Carnegie in 2000. He provided scientific and technical support to the high-pressure programs at the U2A beamline. As the lead beamline scientist of FIS at NSLS-II, the successor of NSLS-U2A, he is in charge the transition of the NSLS-U2A facility to FIS, setting up the Infrared Lab, and the FIS development at NSLS-II. He has a strong background in spectroscopy, solid-state physics, and multiple synchrotron techniques. He actively develops novel experimental techniques (e.g., vacuum far-IR spectroscopy down to 30 cm-1 at high pressure and cryogenic techniques with DACs down to 5 K and 360 GPa, and off-line CO2 laser heating for DACs) and assists users in carrying out research projects in geoscience, planetary science, materials science, condensed matter physics, and chemistry. In addition, he uses synchrotron infrared spectroscopy, Raman scattering, photoluminescence as well as synchrotron x-ray diffraction techniques to explore physical and chemical properties and phase transitions of hydrous minerals, simple molecular systems, and nanoscale materials at high pressures and variable temperatures.

Beamline Operations

Although the user access to the Infrared Lab has not approved due to the rigorous/prolonged safety reviews required by the NSLS-II as the first offline user facility at NSLS-II (see more discussions in the next section), we started to help users through two ways: sample mail-in service and running measurements with users who already got BNL user’s badges. Here is the list of the users groups:

Jie Deng, Yale University, IR-visible measurements of (FeMg)O, (NSF);

Xiang Li, Penn State University, IR measurements of hydrocarbon nanothread, (DOE-Efree);

John Tse, University of Saskatchewan, Canada, (Foreign);

Lingping Kong and Gang Liu, HPSTAR, China, (Foreign).

We expect many more users will request to access and use the facilities in the IR Lab at NSLS-II as soon as NSLS-II, COMPRES, and Geophysical Laboratory have signed the Partner User Agreement.

Beamline Development: Bridging the Gap between NSLS and NSLS-II

The National Synchrotron Light Source (NSLS) had the unique capability to provide synchrotron radiation over a broad range from hard x-ray down to the very far infrared. The infrared radiation at the VUV ring of the NSLS had world-class performance with up to ~104 times the brightness of a conventional thermal (lamp) source.  As one of the six infrared beamlines at the NSLS, U2A was the first dedicated high-P synchrotron infrared facility in the world, with many unique capabilities compared to high-pressure x-ray beamlines. The beamline was built and managed by Geophysical Laboratory, Carnegie Institution of Washington since 1998 and became available for general users in 2000. Through the funding support by COMPRES, the high-pressure infrared program NSLS had been very successful in terms of beamline development, outreach of user community, and scientific productivity. A number of important scientific and technical challenges were being addressed at the facility, making it a highly productive and attractive complement to x-ray sources for high-pressure studies. However, after more than 30 years of service, the NSLS ceased its operation on September 30, 2014. Fortunately, it had been replaced by the NSLS-II, a new state-of-the-art synchrotron facility designed to produce not only x-rays more than 10,000 times brighter than the current NSLS but also more brilliant and stable infrared radiation source. Based on the designed storage ring current of 500 mA, top-up injection and the overall ultra-high beam stability of the facility, our performance estimates indicate that NSLS-II will be world-leading in terms of brightness and signal-to-noise over the widest possible spectral range – including the very far-infrared. This will provide new opportunities for new science under extreme conditions.

In order to continue to accommodate the growing user demand and further expand the high-pressure infrared research at the NSLS-II, we proposed the establishment of Frontier Synchrotron Infrared Spectroscopy Beamline under Extreme Conditions (FIS), an integrated infrared facility dedicated to the study of materials under extreme pressure and temperature (P-T) conditions. The design capabilities of NSLS-II with its small source size, high brightness, and broad spectral range ideally match the materials research under extreme P-T conditions. The continued development of synchrotron IR spectroscopy at NSLS-II will provide superior capabilities for in situ studies under extreme conditions in order to address a broad range of problems in Earth and planetary sciences, physics, biology, chemistry, materials science, and high-technology. The proposal has been approved and NSLS-II is now developing the first beamline to extract infrared beam from one of the special, large-gap dipole bending magnets for FIS/MET. It is one of the five beamlines developed by NSLS-II (BDN) using the operation funds and the IR light will be extracted from the storage ring out to the end stations in early 2018. The beamline hutch, more than four times larger than the U2A hutch at NSLS, is scheduled to complete the construction around the summer of 2016.

Although this is a great opportunity to explore new sciences with a brighter infrared source at NSLS-II, we have to deal with the uncertainties and prolonged gap due to the transition from NSLS to NSLS-II as well as the funding constrains. To bridge the gap and minimize the impact on our user community, two important developments have been made since the NSLS shutdown:

a) The Establishment of an Offline Infrared Laboratory at Bldg. 741, NSLS-II

The NSLS-II management considered all the IR beamlines/programs at

NSLS as a whole and allocated a large laboratory, now called as Infrared Laboratory or IR Lab, at room 1LL10, Bldg. 741 of NSLS-II. After the allocation and the laboratory space became available for beneficial occupancy, we immediately moved and set up the equipment including the Bruker Vertex 80v FTIR spectrometer and Hyperion 2000 IR microscope, the micro-Raman system with full enclosure to comply the laser/lab safety requirements, and the sample preparation working bench. All these have been done before April 2015 and were ready for normal operation with internal IR sources for our COMPRES user community. However, as the first offline user facility at NSLS-II, the safety review process is rigorous and time consuming. Nevertheless, we are making progress and just have lab laser safety document approved (the last safety requirement for the IR Lab). Now we are at the final stage to finalize the Partner User Agreement (PUA) as required by the NSLS-II. We expect the PUA will be signed by the NSLS-II management very soon. That will be the final approval for user’s access to the IR Lab.  Meanwhile we started to offer sample mail-in service or perform experiments together with users who already got new BNL badges (see the list in the section above).

Figure 7. Left: Micro-Raman and FTIR systems installed on the optical table in the IR Lab at Bldg. 741, NSLS-II. Right: 100% line indicating the level of S/N ratio over the mid-IR spectral region through different aperture sizes with a globar IR source.

b) Develop High-Pressure IR Programs at ALS through an Approved AP Proposal

Due to the prolonged dark period for IR programs at NSLS-II, we pursued possibilities to develop user program for the COMPES user community at other synchrotron facilities. After discussed with beamline scientists at Advanced Light Source, we submitted an Approved Program (AP) proposal for developing a high-pressure program at the ALS IR beamlines. The AP proposal has been approved and assured 10% of the total available beam time at beamline 1.4. This will allow us to carry out an extended program of research for a period of up to three years, fully covered the dark period between NSLS and NSLS-II.

A custom IR microscope system for high-pressure experiments has been attached to the existing FTIR spectrometer at beamline 1.4 in August 2015 and preliminary tests look promising in the mid-IR for high spatial resolution. We are going to make an announcement to COMPRES user about this opportunity after the funding support for this activity is finalized by COMPRES.

Figure 8. Left: Custom IR microscope system attached to the Nicolet FTIR spectrometer at beamline 1.4.3, ALS. Right: Mid-IR reflectance spectrum of D2O under high pressure through a 20 m 20 m.

Planned Activities

As the ongoing beamline developments at NSLS-II and new opportunities at ALS, the planned activities for year #5 will include:

a) Routine Offline Infrared Lab Operation at NSLS-II

As the lab safety reviews just have been approved and we are at the final stage to sign a Partner User Agreement (PUA), the Infrared Lab should be ready for users very soon, we are also pushing the NSLS-II user administration office to establish a procedure for users to comply with accessing the IR Lab. We will make an announcement as soon as the PUA signed and BNL site-access instruction for users become available. We have a number of inquiries from users about the IR Lab and expect that majority of the NSLS-U2A users will apply and use the IR facility at the IR Lab for their high-pressure optical studies. It will take at least 50% of the time for the beamline scientist to support the user activities at the IR Lab of NSLS-II in year #5.

b) Planned FIS/MET Beamline Developments/construction

The FY16 is crucial for FIS/MET beamline developments including major procurements (>$1M) of the dipole vacuum chamber for IR beam extraction and acquisition/installation of the FIS/MET hutches (~$800K) under the best funding scenario. If the requested NSLS-II operation funds secured, the FIS/MET beamline hutches are expected to be constructed and ready for beneficial occupancy in the summer of 2016. Therefor, we request $80K equipment funds (originally approved in our five year budget) to purchase a Vertex 70v FTIR spectrometer from BrukerOptics. This new spectrometer is needed to replace the Bruker IFS 66v spectrometer (over 20 years old) and install at the new FIS hutch as the first end station. This will allow us to keep the routine IR Lab operation and go through all the rigorous and time consuming safety reviews required by the NSLS-II after the equipment installations inside the hutch. It is crucial to ensure the beamline is ready on day one when the synchrotron IR beams to be extracted and delivered to the hutch. The equipment at the IR Lab will be relocated to FIS hutch as the second end station.

c) Explore New Opportunity at ALS

The initial set up and test of the custom IR microscope system clearly demonstrated the feasibility of the beamline 1.4 in the mid-IR spectral range at ALS for high-pressure experiments. However, there is also a clear cut-off for the far-IR region below 300 cm-1 due to the dipole chamber. Recently, we learnt that there is a new IR beamline under construction at ALS and has a potential opportunity to modify the dipole chamber with large vertical opening. A conceptual calculation predict a significantly better far-IR performance without attenuation down to 20 cm-1. As the ALS vacuum chamber was actually in the original designs but wasn't implemented for some lost-to-time reason, this opens a window for us to promote the idea to modify the vacuum chamber for much better far-IR beam extraction. The timing is also important because the beamline construction is scheduled to complete less than one year. We are looking for varies options and continuing to explore such opportunity. If ALS management decides to provide funds for the chamber modification and the IR beam extraction takes less than a year, we propose to pursue a dedicated high pressure IR facility at ALS by contributing the Vertex 70v FTIR spectrometer together with the custom IR microscope and a micro-Raman system for the beamline. This will significantly shorten the “dark period” caused by the transition from NSLS to NSLS-II.

Budget

Actual and anticipated spending for beamline operations (June 2015– May 2016):

1.  Budget for Beamline Operations:  June 2015-May 2016

| Personal: Beamline scientist | Salary plus fringe benefits | $144,144 | These include a normal raise and cost of living adjustment for the beamline scientist. |
| --- | --- | --- | --- |
| Materials/supplies | Operation | $2,000 | Miscellaneous expenses for the offline beamline operation at NSLS-II |
| Travel | Travel for beamline scientist Travel for ALS | $1,000 $2,000 | Travel expense for the beamline scientist to attend COMPRES annual meeting. Travel expenses to develop high-pressure IR program and support users for their experiments at ALS. |
|  | Total direct cost | $149,144 |  |
| Total cost for operation |  | $195,379 | This total includes indirect costs at a rate of 31%. ($149,1441.31*) |

2.  Budget request for IR facility Operations:  June 2016-May 2017

| Personal: Beamline scientist | Salary plus fringe benefits | $149,910 | These include a normal raise and cost of living adjustment for the beamline scientist. |
| --- | --- | --- | --- |
| Materials/supplies a. b. | Operation Operation | $10,016 $8,000 $10,000 | Miscellaneous expenses for the offline Infrared Lab operation at NSLS-II; Miscellaneous expenses for high-pressure IR beamline activities/user experiments at ALS (4$2000/trip); Miscellaneous expenses for relocating and installation of the end station equipment at the FIS hutch on the NSLS-II experimental floor. |
| Travel | Travel for beamline scientists | $3,000 | Travel between BNL and Washington as well as for the beamline scientist to attend COMPRES and other scientific meetings. |
|  | Total direct cost | $180,916 |  |
| Total cost for operation |  | $237,000 | This total includes indirect costs at a rate of 26%. ($180,9161.31*) |

###### Budget for Capital Equipment

| Vertex 70v FTIR spectrometer | BrukerOptics | $80,000** | New FTIR spectrometer to replace the IFS 66v (more than 20 years old) spectrometer for second end station at FIS, NSLS-II or installed at ALS depending on the progress. |
| --- | --- | --- | --- |
| Total request for equipment |  | $80,000 |  |

* Indirect Costs
Carnegie has a rate agreement with its cognizant agency, NSF, which has issued a maximum provisional indirect cost rate of 75.0% for FY2015; however, for administrative purposes the Carnegie Institution is voluntary limiting its indirect cost recovery to 31.0% of Total Modified Direct Costs (TMDC). Total Modified Direct Costs are calculated as Total Direct Costs minus Equipment, Participant Support Costs, and Subcontract costs in excess of $25,000 for each subcontract.

** The quoted price of the FTIR spectrometer is $101,180. Geophysical Laboratory will cover the rest cost if this funding request is approved.

Appendices

1. A publication list for 2014 and 2015.

Boulard, E., D. Pan, G. Galli, Z. Liu, W. Mao, Tetrahedrally coordinated carbonates in Earth’s lower mantle, Nat. Commun., 6, 6311 (2015).

Brinzari, T., K. O'Neal, J. Manson, J. Schlueter, A. Litvinchuk,  Z. Liu,  and J. L. Musfeldt, Local lattice distortions in Mn[N(CN)2]2 under pressure, Inorganic Chem., submitted.

Chellappa, R., D. Dattelbaum, J. Coe, N. Velisavljevic, L. Stevens, Z. Liu, Intermolecular Stabilization of 3,3′-Diamino-4,4′-azoxyfurazan (DAAF) Compressed to 20 GPa, J. Phys. Chem. A, 118(31), 5969-5982 (2014).

Chellappa, R., D. Dattelbaum, N. Mack, T. Ahmed, M. Graf, A. Azad, D. Moore, Z. Liu, Vibrational Anharmonicity in 1,3-dinitrobenzene, J. Phys. Chem. A., submitted

Cui, J., M. Yao, H. Yang, Z. Liu, F. Ma, Q. Li, R. Liu, B. Zou, Z. Liu, and B. Liu, Structural Deformation of Sm@C88 Under High Pressure, Sci. Rep., 5, 13398 (2015).

Farahi, N., S. Prabhudev, G. Botton, J. Zhao, J. Tse, Z. Liu, J. Salvador, H. Kleinke, Local Structure and Thermoelectric Properties of Mg2Si0.977-xGexBi0.023 (0.1 ≤ x ≤ 0.4), J. Alloys Compd., 644, 249-255 (2015).

Jaffe, A., Y. Lin, W. Mao, H. Karunadasa, Pressure-Induced Conductivity and Yellow-to-Black Piezochromism in a Layered Cu–Cl Hybrid Perovskite, J. Am. Chem. Soc., 137(4), 1673-1678 (2015).

Li, D., F. Li, Y. Li, X. Wu, G. Fu, Z. Liu, X. Wang, Q. Cui, H. Zhu, High-pressure Studies of Rubidium Azide by Raman and Infrared Spectroscopies, J. Phys. Chem. C, 119(29), 16870-16878 (2015).

Liu, G., L. Kong1, J. Yan, Z. Liu, H. Zhang, H, Mao, B. Chen, Nanocrystals: Controlling the Interior from the Surface, Phys. Rev. Lett., submitted.

Ma, Y., Z. Liu, A. Geng, T. Vogt, Y. Lee, Structural and Spectroscopic Studies of Alkali-metal Exchanged Stilbites, Microporous and Mesoporous Materials, submitted.

O’Neal, K., J. Cherian, A. Zak, R. Tenne, Z. Liu, and J. Musfeldt, Revealing the pressure-induced breakdown pathway in WS2 nanotubes, Nano. Lett., submitted.

Panero, W., J. Pigott, D. Reaman, J. Kabbes, Z. Liu, Dry (Mg,Fe)SiO3 perovskite in the Earth’s lower mantle, J. Geophys. Res., 120(3), 894–908 (2015).

Thompson, E., A. Campbell, Z. Liu, In-situ infrared spectroscopic studies of hydroxyl in amphiboles at high pressure, American Mineralogist, accepted.

Wang, Y., J. Zhu, W. Yang, T. Wen, M. Pravica1, Z. Liu, M. Hou, Y. Fei, L. Kang, Z. Lin,

C. Jin, and Y. Zhao, Hidden Structure Memory Effect in Pressure-Amorphized VO2(B) Nanosheets, Nat. Commun., submitted.

Wu, T., T. Tyson, H. Chen, P. Gao, T. Yu, Z.  Chen, Z.  Liu, K. Ahn, X. Wang, and S.W. Cheong, Pressure Dependent Structural Changes and Predicted Electrical Polarization in Perovskite RMnO3, J. Phys.: Condens. Matter, submitted.

Zhao, J., Z. Liu, J. Reid, K. Takarabe, T. Lida, B. Wang, U. Yoshiya, J. Tse, Thermoelectric and electrical transport properties of Mg2Si multi-doped with Sb, Al and Zn, J. Mater. Chem. A, 3, 19774-19782 (2015).

Zhao, J., Z. Liu, R. Gordon, K. Takarabe, J. Reid, and J. Tse, Pressure-induced phase transition and electrical properties of thermoelectric Al-doped Mg2Si, J. Appl. Phys., 118, 145902

(2015).

Zhao, Z., H. Zhang, H. Yuan, S. Wang, Y. Lin, Q. Zeng, G. Xu, Z. Liu, G. Solanki, et al., Pressure Induced Metallization with Absence of Structural Transition in Layered Molybdenum Diselenide, Nat. Commun., 6, 7312 (2015).

Zhu, J., Z. Quan, Y.S. Lin, Y.B. Jiang, Z. Wang, J. Zhang, C. Jin, Y. Zhao, Z. Liu, C.J. Brinker, H. Xu, Porous Ice Phases with VI and Distorted VII Structures Constrained in Nanoporous Silica, Nano Lett., 14(11), 6554–6558 (2014).