# COMPRES Multi-Anvil Synchrotron Program

2018 COMPRES Annual Report

November 2017 – November 2018

Prepared by Donald J. Weidner, Matthew L. Whitaker, Haiyan Chen

Mineral Physics Institute, Stony Brook University, Stony Brook, NY 11794-2100

## Table of Contents

COMPRES Multi-Anvil Synchrotron Program	1

Table of Contents	1

Overview	2

Scientific Highlights	4

Beamline Personnel	15

Beamline Operations	15

Beamline Community Activities	17

Beamline Development	18

Budget	22

Budget Justification	23

User Satisfaction	24

Appendix 1.  Publications	25

Appendix 4.  Workshop Report	29

Apppendix 5.  End of Run responses	37

## Overview

Here we present the annual report for the combination of beamlines 6BMB at the APS and MAXPD at the NSLS II. The current facility (including 6BMB and MAXPD) represents the leading edge for mechanical studies in Earth sciences at this time.  While we were the developers of the first high pressure, synchrotron based, deformation machine (the DDIA), many other synchrotrons have adapted this system.  However, we are the only beamline program using the RDA (rotational Drickamer apparatus), which can reach lower mantle conditions and produce very high sample strains, and we are also the only beamline using the DT25 which should also reach lower mantle conditions.  We are the only deformation program using a conical slit for the cleanest diffraction patterns for white-beam stress measurements. We have the first ultrasonic system capable of measurements in lab times of less than one second.  We are the first lab program measuring elastic moduli and Q at frequencies between 1 Hz and 1 mHz (Spring8 has recently adapted our technique) into the multiple GPa range.  In all of these cases, our designs have been published and other synchrotron facilities can and often do copy them. Our position in the field results from continued enhancement of experimental capabilities and is driven by the user community.

The COMPRES Multi-Anvil Synchrotron Program currently provides a white beam for the DDIA and RDA at 6BMB of APS and a monochromatic beam for the new DT25 at MAXPD at the NSLS-II.  The total beamtime will be similar if not more than we had at the NSLS-I (66%), and the total beamline scientist support from COMPRES will be the same as at the NSLS-I.  Our program is divided between two beamlines in much the same manner as the multi-anvil program at GSECARS, where one beamline provides a stable platform for many experiments (6BMB) and the other uses a world class beam (MAXPD) with a unique high pressure apparatus (DT25) allowing a new range of experiments.  While the responsibilities of the beamline scientists are mostly divided with Haiyan Chen at APS and Matthew Whitaker at NSLS-II, in fact, Whitaker still spends about 25% of his time at APS assisting users and working to maintain the 6BMB system

The 6-BM-B Beamline at the Advanced Photon Source (APS), Argonne National Laboratory, is a bending magnet "white beam" beamline (~20-100 keV) utilizing energy dispersive diffraction (EDD) and X-radiographic imaging techniques in mineral physics and Earth materials studies. This beamline is a dedicated high pressure multi-anvil facility equipped with a 250-ton hydraulic press with DIA, Deformation-DIA, and T-cup pressure modules, a Canberra 10-element Ge solid state detector array with conical slit system, and a Prosilica CCD camera for X-radiographic imaging. This beamline provides experimental capabilities for studying hydrostatic and dynamic phase transformations, equations-of-state, melting processes, steady-state and dynamic rheological properties, as well as transport, thermal, elastic, and acoustic properties to COMPRES affiliated users in United States and abroad.

We have completed building an endstation in hutch D of the XPD beamline at the NSLS II.  XPD-D is located on an insertion device beamline at NSLS-II, designated 28-ID-2-D, and is served by a 1.8 Tesla Damping Wiggler source. This beamline is designed to make use of monochromatic X-ray beam, which is tunable in energy from ~30-70 keV. The most common energies used for operations are 52 and 67 keV. This high energy will be advantageous since it is very penetrative and the diffraction signal will cover a small two-theta region, yielding more diffraction rings in a limited two-theta aperture.  We use a Perkin-Elmer (1621 CN3 EHS) flat panel detector, CsI phosphor directly deposited on amorphous silicon, 2048x2048 pixels with 200x200 micrometer pixel size, 15 frames per second readout at 2048x2048, and 30 fps at 1024x1024. We also collect X-radiographic imaging of the sample during experiments by focusing a Point Gray CCD camera on a YAG scintillator.

In the MAXPD (Multi-Anvil X-ray Powder Diffraction) endstation, we have installed a 1100-ton hydraulic press designed for high pressure and high temperature multi-anvil experiments. The standard pressure module that will be in use in this press is the DT-25, which is a differential Kawaii-type multi-anvil system that is capable of achieving pressures ranging from the crust through the transition zone to lower mantle pressures. This module is the first of its kind in the world, and permits new experiments not possible at any other beamline. A more standard D-DIA module is also available for use in this press. Both the DT-25 and the D-DIA have separately actuated top and bottom anvils that are driven by separate differential hydraulic pumps to allow studies involving differential stress and strain and sample deformation mechanisms.

## Scientific Highlights

Simultaneous mHz and MHz Elastic Moduli Measurements at High P & T

Donald J. Weidner, Li Li, Matthew L. Whitaker, Haiyan Chen, and Richard Triplett

Mineral Physics Institute, Stony Brook University

We present an experimental protocol using a DDIA at a synchrotron that simultaneously yields Young’s modulus at mHz frequencies and the bulk and shear moduli at MHz frequencies.  Mineral physics has gathered a vast library of elastic properties for minerals that allow calculations of P and S velocities along with elastic anisotropy at temperatures and pressures that span those of the Earth.  However, most of these results are obtained at MHz frequencies or higher using ultrasound, Brillouin spectroscopy, or density functional theory.  However, seismology determines these properties at Hz to mHz frequencies for the Earth’s interior.  In most cases mHz and Mhz elastic properties are the same.  However, some processes can disperse acoustic waves yielding a frequency dependence in the acoustic velocities and attendant attenuation.  Here we demonstrate a new experimental protocol that yields simultaneous measurements of mHz and MHz elastic properties at the COMPRES beamline, 6BMB at the APS.  The high frequency data are obtained from ultrasonic measurements using the DIASCoPE probe while low frequency (mHz) data are obtained from stress/strain measurements with forced oscillations. Together, amplitude and phase yield Young’s modulus and Q.  Examples of data obtained will be presented.

Low-temperature plasticity in olivine: Grain size, strain hardening, and the strength of the lithosphere

Lars. N. Hansen, Christopher A. Thom, Kathryn M. Kumamoto, David Wallis, William B. Durham, David L. Goldsby, Thomas Breithaupt, David L. Kohlstedt

Department of Earth Sciences, University of Oxford, Oxford, United Kingdom

Submitted to Journal of Geophysical Research: Solid Earth

Plastic deformation of olivine at relatively low temperatures (i.e., low-temperature plasticity) likely controls the strength of the lithospheric mantle in a variety of geodynamic contexts.  Unfortunately, laboratory estimates of the strength of olivine deforming by low-temperature plasticity vary considerably  from study to study, limiting confidence in extrapolation to geological conditions. Recent work suggests that a grain-size dependence or strain-dependence of strength could explain the discrepancies among previous studies.  Here we present the results of a set of deformation experiments on olivine aggregates with mean grain sizes of 3 to 700 μm. Sample material was obtained from hot-pressed San Carlos olivine, natural dunites, and single crystals of San Carlos olivine. Deformation experiments were conducted in a deformation-DIA at confining pressures of 5 to 9 GPa and temperatures of 298 to 1473 K. Most assemblies consisted of two samples of different grain size stacked one on top of the other and separated by Ni foil. Several assemblies were constructed with single samples for comparison to  two-sample assemblies. Assemblies were deformed in triaxial compression with the lengths of each sample monitored using synchrotron radiographs. Stresses were determined  using energy-dispersive X-ray diffraction. Samples were either shortened unidirectionally or cyclically shortened and lengthened. The results of these experiments demonstrate that, under conditions in which low-temperature plasticity is the dominant deformation mechanism, fine-grained samples are initially stronger than coarse-grained samples. Radiography clearly demonstrated that coarse-grained samples yielded at lower stresses than their fine-grained counterparts in the same assembly. Yield stress also decreased with increasing temperature. All samples exhibited significant strain hardening until an approximately constant flow stress was reached. The magnitude of hardening was independent of grain size and temperature. Cyclical experiments revealed a Bauschinger effect, in which the initial yield strength is higher than the yield strength during subsequent cycles. Both the hardening and Bauschinger effect are interpreted to result from the development of back stresses associated with long-range dislocation interactions.  We calibrated a constitutive model based on these observations that accounts for the roles of grain size and plastic strain in moderating the plastic strength of olivine. Extrapolation of this calibrated model to geological conditions predicts that the strength of the lithosphere at yield is low relative to previous experimental predictions but increases significantly with increasing strain. Our results resolve apparent discrepancies in recent observational estimates of the strength of oceanic lithosphere.

Bulk modulus of Fe-rich olivines corrected for non hydrostaticity

Frederic Bejina; Misha Bystricky; Nicolas Tercé; Matthew L Whitaker; Haiyan Chen

IRAP, Observatoire Midi-Pyr´en´ees, Toulouse, France

Comptes Rendus Geoscience, https://doi.org/10.1016/j.crte.2018.06.002

In situ X-ray diffraction was used to measure the isothermal bulk modulus K_T0 of synthetic olivines with different iron contents. The chemical formulae of the olivine samples were [Fe_xMg_(1-x)]2SiO_4 with x=0.44, 0.63 0.81 and1 (referenced as Fa44, Fa63, Fa81 and Fa100 respectively). All experiments were performed in the multi-anvil apparatus installed at NSLS beamline X17B2, to pressures up to about 7 GPa. Unit-cell volumes under hydrostatic conditions and differential stresses present in the samples were calculated using the method developed by Singh et a. (1998), and pressures measured using NaCl as a standard were then corrected for these stresses. Using a second-order Birch-Murnaghan equation of state, we obtained the isothermal bulk modulus of each composition: K_T0^Fa44 = 132.3 GPa, K_T0^Fa63 = 132.1 GPa, K_T0^Fa81 = 136.3 GPa and K_T0^Fa100 = 134.8 GPa. These values combined with data available in the literature show that K_T0 of Fe-rich olivines increases very slowly with Fe content.

EOS study of the effect of ferric iron on the EOS of Ahrensite

Liping Wang, Pei Wang et al

University of Nevada Las Vegas. Las Vegas, USA

The physical properties of the high-pressure polymorphs of olivine have major implications for the structural and chemical makeups of the mantle transition zone. Yet the effect of minor ferric iron on those properties is unknown. EOS study of the effect of ferric iron on the EOS of Ahrensite was performed at APS 6BM-B Beamline. Starting sample is fayalite plus 5% magnetite. At high P and T, the mixture was converted to gamma phase, the Ahrensite. High quality data have been collected using multi-element detector at 6-BMB. The slide shows data from two detectors-- vertical and horizontal ones, and two data sets agree very well, indicating hydrostatic and uniform pressure. The data were also collected on pure Ahrensite for comparison. The EOS analyses are under way.

Note: Elastic wave velocity measurement using ultrasonic system with two-reflectors

L. Li, M. L. Whitaker, and D. J. Weidner

Mineral Physics Institute, Stony Brook University, Stony Brook, NY, USA

Review of Scientific Instruments, 89 (8) (2018) Article # 086105

We report an experiment protocol for ultrasonic velocity measurements carried out in a synchrotron-based multi-anvil high-pressure apparatus. By adding a second reflector in the cell assembly, ultrasonic signals are significantly stabilized for experiments where samples experience structural changes such as phase transition or partial melting. Features of the elastic wave can be calculated from acoustic impedance allowing us to evaluate the possible interference between the sample reflection and other internal multiple reflections. This new protocol allows various forms of samples including cold-pressed powder samples, for which grain size and porosity can be monitored and controlled in situ during the annealing process.

In-situ Deformation of Olivine during Phase Transformation to Ringwoodite

Anwar Mohiuddin, Jennifer Girard and Shun-ichiro Karato

Department of Geology and Geophysics, Yale University, New Haven, USA

Submitted to Nature Geoscience

Rheological properties govern the nature of material deformation in subducting slabs and the surrounding mantle Grain-size of the constituent phases and their spatial distribution changes during phase transformations and this has a leading order effect on rheological properties of materials. Hence an understanding of changes in the grainscale microstructure and rheological properties of the constituent phases during phase transformation is essential to understand the deformation of a composite. The only way to quantify rheological properties of constituent phases in a composite during its deformation is to conduct in-situ experiments using X-ray diffraction at a synchrotron facility. We use Rotational Drickamer Apparatus (RDA) and take advantage of the white X-ray beam at Argonne National Laboratory to collect X-ray diffraction pattern. We have reached the following coclusions:

1. Stress required to deform ringwoodite that transformed from olivine is less than the stress required to deform its parent (olivine) and pre-synthesized ringwoodite. 2. Stress required to deform ringwoodite that transformed from olivine increases with time whereas pre-transformed ringwoodite does not show hardening. 3. Stress data from ringwoodite that transformed from olivine shows no (hkl) dependence, whereas pre-transformed (coarse grained) ringwoodite does show (hkl) dependence. Using the theory of Karato (2009), we conclude that pre-transformed ringwoodite deforms by dislocation creep whereas ringwoodite transformed from olivine deforms by diffusion creep. This conclusion is also consistent with the presence/absence of time hardening.

Ultrasonic Acoustic Wave Velocities of Neighborite (NaMgF3) Across Orthorhombic to Cubic Phase Boundary at High P-T

Li Li, Donald J. Weidner, Matthew L. Whitaker, Richard Triplett

Mineral Physics Institute, Stony Brook University, Stony Brook, NY, USA

Physics of the Earth and Planetary Interiors, 283 (2018) 38-42

Neighborite perovskite (NaMgF3) is a close analogue to Mg or Ca silicate perovskite. It experiences a second-order phase transition at high T that may have a significant effect on the acoustic velocities. Here we report the measurement of elastic wave velocities of neighborite perovskite using the multi-anvil high-pressure apparatus located in beamline 6-BM-B at the Advanced Photon Source. We use the newly installed ultrasonic interferometry equipment using pulse-echo-overlap method coupled with the D-DIA device. X- radiographic imaging is used to measure sample length at high P-T. X-ray diffraction spectrum is used to determine the pressure and sample conditions. Precise measurements of P and S wave velocities are at 60 and 35 MHz respectively and are nearly simultaneous. We use a double reflector method to enable measurement of elastic wave velocities of cold-pressed polycrystalline sample that is sintered in situ at high P-T. Experiments were carried out up to 3 GPa and 1000º C. Our results indicated that MHz ultrasonic acoustic velocities experience a significant minimum at the phase transition,

with the bulk modulus being more affected than the shear modulus. The shear modulus of the cubic phase appears unchanged from those of the orthorhombic phase, while the bulk modulus increases in transforming into the cubic phase.

Making tissintite: Mimicking meteorites in the multi-anvil

Melinda J. Rucks, Matthew L. Whitaker, Timothy D. Glotch, John B. Parise, Steven J. Jaret

Tristan Catalano, M. Darby Dyar

Stony Brook University, Stony Brook, NY, USA

American Mineralogist (2018) 103 (9): 1516-1519

Tissintite is a shock-induced, Ca-rich mineral, isostructural to jadeite, observed in several meteorite samples such as the martian shergottite Tissint. It may form within a “Goldilocks Zone,” indicating a potential to provide strict constraints on peak pressure and temperature conditions experienced during impact. Here we present the first laboratory synthesis of tissintite, which was synthesized using a large volume multi-anvil apparatus at conditions ranging from 6–8.5 GPa and 1000–1350 °C. For these experiments, we utilized a novel heating protocol in which we reached impact-relevant temperatures within 1 s and in doing so approximated the temperature-time conditions in a post-shock melt. We have established that heating for impact-relevant timescales is not sufficient to completely transform crystalline labradorite to tissintite at these pressures. Our findings suggest that tissintite forms from amorphous plagioclase during decompression.

Grain Twinning Mediated by Shear-Induced Phase Transition at Grain Boundary in Microcrystalline cBN

Pei Wang, et al

University of Nevada at Las Vegas, Las Vegas, USA

Understanding the plastic deformation and phase transformation mechanisms of materials under high shear stress and high temperature are of crucial importance to both materials science and technological applications. The deformation behavior of superhard cubic boron nitride (cBN), important to fundamental physics and industrial applications, has been extensively investigated by theoretical simulations at 0 K and experiments under uncontrolled stress conditions. However, very little research has been done to elucidate the plastic deformation mechanisms of cBN because of the experimental obstacles to deform superhard materials with real time control of stress conditions. To overcome this challenge, we performed the experimental deformation studies of microcrystalline cBN using synchrotron-based in situ energy dispersive x-ray diffraction and radiographic imaging measurements at high pressure and temperature in a D-DIA large volume deformation apparatus to reveal the strain-stress relations during deformation. Elastically and plastically deformed cBN samples are then imaged by High-resolution transmission electron microscopy (HRTEM) for the structural characterization at atomic level.

Pressure Dependence of Magnesite Flow Strength

Millard, J. W., McDaniel, C. A., Holyoke, C.W., Raterron, P., Kronenberg, A.K., Tokle, L

University of Akron, Akron, OH, USA

Recent experiments by Holyoke et al. (2014) indicate that magnesite is weak compared with olivine, leading to strain localization into magnesite-bearing horizons of downgoing subducting slabs, potentially leading to intermediate (200-400 km) deep focus earthquakes DFEs. Flow strengths (σ) of magnesite were determined as functions of strain rate and temperature (T), but not of pressure (P). In order to determine the pressure dependence of magnesite deforming by dislocation creep and low temperature plasticity (LTP), we performed triaxial deformation experiments on natural, coarse-grained (80 μm) magnesite aggregates at T = 900oC, strain rate of 2*10-5/s and P from 3.2 to 7.9 GPa in the D-DIA at Beamline 6-BMB at the Advanced Photon Source at Argonne National Lab. Strengths of magnesite aggregates increase as a function of pressure, from σ=0.85 GPa (at P = 3.2 GPa) to 1.69 GPa (at P = 7.9 GPa). Microstructures developed at P ≤ 6.2 GPa include grain flattening and sweeping undulatory extinction, and dynamic recrystallization at grain boundaries (by bulge nucleation), consistent with dislocation creep. Microstructures generated at P ≥ 6.7 GPa include some twins and kink bands, grain flattening and patchy undulatory extinction, and bulging recrystallization at grain boundaries. These microstructures indicate contributions to strain of both LTP and dislocation creep mechanisms. The pressure dependence can be described by a thermally activated enthalpy, where s increases with P by way of an activation volume V*. Based on both D-DIA and Griggs apparatus results (of Holyoke et al., 2014), V* = 8.5 (± 0.5) × 10-6 m3 mol-1. This value is comparable to V* determined for creep of olivine (9.5 (± 7) × 10-6 m3 mol-1 Durham et al., 2009), indicating that the strength difference between magnesite and olivine will continue to be significant at high pressures, and lead to strain localization at all subduction zone depths.

ULTRASONIC P AND S WAVE VELOCITY MEASUREMENTS ON POLYCRYSTALLINE OLIVINE DURING DEFORMATION USING THE

DIASCoPE AND D-DIA APPARATUS AT APS 6BMB

Pamela Burnley, Matthew L. Whitaker, Taryn Traylor, Shirin Kaboli, Dawn Reynoso

Department of Geoscience, University of Nevada, Las Vegas, NV

The development of the DIASCoPE which can collect ultrasonic spectra in under a second allows ultrasonic measurements to be incorporated into a standard in-situ deformation experiment in the D-DIA apparatus. This instrumentation is installed at the COMPRES supported 6BMB beamline at the Advanced Photon Source. To facilitate such experiments we developed a hybrid D-DIA sample assembly with characteristics of both an ultrasonic assembly and a deformation assembly. Using this cell we conducted a suite of low strain experiments on polycrystalline San Carlos olivine. The olivine was pre-sintered at 1150 C at 300 MPa, cored to produce a right cylinder and then polished so as to produce perfectly parallel ends. The olivine was assembled in series with an alumina buffer rod and another fully dense alumina polycrystal (‘the piston’) with gold foils on the interfaces. The sample was deformed at 3 GPa at a strain rate of ~5 x 10-6/sec at 450, 650 and 850 C. We measured diffraction from the sample at 12 minute intervals, measured diffraction from the alumina piston at 12 minute intervals, and collected P- and S- wave scans and radiographic images every 6 minutes. For each deformation sequence we fit the diffraction data using EPSC models to obtain the load supported by the sample. Sample lengths were determined from the radiographs and fit with a polynomial as a function of time within each sequence to produce a smooth sample length vs time curve. Travel times were determined using a fitting routine in Plot85 and velocities were calculated using the smoothed sample length curves.  Preliminary analysis of the data indicate that P and S wave velocities change in response to loading in different ways with P wave velocities changing proportional to compressive stress but the S-wave velocities being more strongly influenced by the onset of ductile deformation.

## Beamline Personnel

Donald J. Weidner – Program P.I.					0% C / 100% SBU / 0.5 FTE

Matthew L. Whitaker – Co-P.I. & Beamline Scientist 		100% C / 0% SBU / 1.0 FTE

Haiyan Chen – Beamline Scientist					100% C / 0% SBU / 1.0 FTE

Michael T. Vaughan – Scientific Staff Associate			0% C / 100% SBU / 0.7 FTE

Kenneth J. Baldwin – Software and Controls Specialist		0% C / 100% SBU / 0.8 FTE

William R. Huebsch – Electronics Design and Support		25% C / 75% SBU / 0.8 FTE

Samantha Lin – Administrative Support				0% C / 100% SBU / 0.3 FTE

Richard S. Triplett – Graduate Student				0% C / 100% SBU / 0.5 FTE

Li Li, Research Associate Professor					0%C, / 100% SBU,0.15 FTE

## Beamline Operations

One shift of beamtime at both NSLS-II and APS is 8 hours. Both 6-BM-B and XPD-D are on a shared beamline where only one endstation can be active at a time, so there is no parasitic or tandem mode availability. The performance metrics for both 6-BM-B and XPD-D are found in the attached Excel file and included as Appendices 2 and 3.

APS 6-BM-B:

In the 3 cycles of 2017-2018 (2017-3, 2018-1, 2018-2), 6BM-B had 326 shifts, ~52% from APS, of which 285 shifts, or 87.4% were available to users. Over-subscription rate is 131%. COMPRES assigned ~66.3% of beamtime and APS assigned ~31.6% of beamtime to users from COMPRES member institutes. Among the 44 proposals received, 84% are earth science related proposals. Of the 38 proposals granted beamtime, 84.2% were in earth science. NSF-EAR funded 61.4% of all proposals granted beamtime.

This is compared with the 3 cycles of 2016-2017 (2016-3, 2017-1, 2017-2), when 6-BM-B had 322 shifts, ~52% from APS, of which 295 shifts, or 91.6% were available to users. Over-subscription rate is 127%. COMPRES assigned ~64% of beamtime and APS assigned ~34% of beamtime to users from COMPRES member institutes. Among the 41 proposals received, 87.8% were earth science related proposals. Of the 36 proposals granted beamtime, 86.1% were in earth science. NSF-EAR funded 65% of all proposals granted beamtime.

6-BM-B Beamtime Statistics 2018 (2017 in Parentheses):

Number of beamtime proposals received: 44 (41)

Number of beamtime proposals granted beamtime: 38 (36)

Total number of shifts requested: 375 (372)

Total number of shifts granted: 285 (295)

Total number of shifts available: 285 (295)

Oversubscription rate (= shifts requested / shifts available): 131% (127%)

Number of visits by distinct research groups: 90 (59)

Number of unique users, categorized by affiliation:

Total Unique Users: 29 (42)

Academic Institutions: 28 (42)

Industry: 1 (0)

USA: 24 (35)

Europe: 4 (7)

China: 1 (0)

Total number of person-visits: 110 (99)

Number of undergraduate users: 0 (3)

Number of graduate student users: 14 (14)

Number of visits funded by each funding agency

NSF: 78

DOE/NNSA: 16

NASA: 6

Foreign: 9

6-BM-B Performance Metrics 2018:

General User Time for COMPRES: For higher-ranked proposals with beamtime allocated by the facility: 31.6%

Contributing User Time: Allocated for COMPRES users who were not assigned GU time: 66.3%

Total time for COMPRES users (there will usually be GU and CU COMPRES users): 100% (All users are from COMPRES member institutes)

MAXPD Overview and Performance Metrics:

The last year has seen several exciting developments at the MAXPD beamline. Installation and commissioning has all but completed, and the endstation has been officially added to the General User program for the XPD beamline. XPD management has been very accommodating in providing valuable beamtime for us to commission and optimize our installation. Discussion of the tasks and results of beamtimes over the last year are found under Beamline Development.

At present, there still has yet to be a formal Partner User Agreement filed between COMPRES and NSLS-II management. At the request of the NSLS-II management, a new Partner User Proposal was submitted on May 31, 2017 to supersede the terms of the previous PUP that was rendered invalid by the discontinuation of the COMPRES Diamond Anvil Cell program at NSLS-II. The proposal has been reviewed by management, and the process and negotiations remain ongoing.

In the 3 cycles of 2018 (2018-1, 2018-2, 2018-3), MAXPD had 59 shifts, ~11% of total NSLS-II available shifts. All beamtime was subjected to the General User Proposal system, and 44 of these 59 shifts were granted GU time by the Beamtime Allocation Committee (~75%). The 2018-3 cycle is the first time we are being granted CU time, despite the Partner User Agreement not being finalized. COMPRES assigned ~25% of beamtime and NSLS-II assigned ~75% of beamtime to users from COMPRES member institutes. Subscription rate for 2018 was 173%. Among the 13 proposals received, 84.6% were earth science related proposals. Of the 9 proposals granted beamtime, 100% were in earth science. NSF-EAR funded 66.7% of all proposals granted beamtime.

This is compared with the 3 cycles of 2017 (2017-1, 2017-2, 2017-3), when MAXPD had 26 shifts, ~5% of total NSLS-II available shifts. Most of this time was Technical and Science Commissioning time, but all Science Commissioning beamtime was subjected to the General User Proposal system, and 8 of these 26 shifts were granted GU time by the Beamtime Allocation Committee (~31%). Subscription rate for 2017 was 100%. Among the 1 proposal received, 100% were earth science related proposals. Of the 1 proposal granted beamtime, 100% were in earth science. NSF-EAR funded 100% of all proposals granted beamtime.

MAXPD Beamtime Statistics 2018 (2017 in Parentheses):

Number of beamtime proposals received: 13 (1)

Number of beamtime proposals granted beamtime: 9 (1)

Total number of shifts requested: 102 (6)

Total number of shifts granted: 59 (8)

Total number of shifts available: 59 (8)

Oversubscription rate (= shifts requested / shifts available): 1.73 (1.0)

Number of visits by distinct research groups: 10 (1)

Number of unique users, categorized by affiliation (University, Gov’t Lab/agency, Private institution, or Industry) and by origin (USA, Canada, Europe, Asia, Other):

Total Unique Users: 17 (11)

Stony Brook University: 9 (8)

NSLS-II: 5 (3)

Yale University: 2 (0)

University of Lille: 1 (0)

USA: 16 (11)

France: 1 (0)

Total number of person-visits: 28 (11) (Not Counting NSLS-II Staff)

Number of undergraduate users: 0 (0)

Number of graduate student users: 4 (1)

Number of visits funded by each funding agency:

NSF: 23 (11)

NASA: 3 (0)

Foreign: 2 (0)

## Beamline Community Activities

- We held a successful COMPRES-sponsored workshop on future directions for large volume synchrotron-based research in late September titled: Envisioning the Next Generation of In-situ Synchrotron X-ray Techniques in Large-Volume High Pressure Apparatus for Mineral and Rock Physics. There were 59 attendees representing rock physics, mineral physics, and synchrotron physics.  The workshop explored 4-D renderings of the sample (including time) with diffraction and imaging.  We feel that the workshop was very successful, bringing a large number of scientists that have not been involved with synchrotron research into the discussion.  One very promising aspect of the workshop is the fact that rock physics community formed a working group to prepare a proposal for a Research Coordination Network (NSF) focused on continuing such workshops over the next five years (up to $500k) to allow rock physics to exploit the power of the synchrotron in the research programs.  See an expanded description of the workshop in Appendix 4.

- Experiments making use of the new technical developments led by Whitaker at 6-BM-B, including the DIASCoPE and planetary impact/petrology experiments, now account for half of the proposal pressure at the beamline.

## Beamline Development

MAXPD –  DT25

The DT25 is a Kawai-type multi-anvil system designed for use in deformation experiments. The Kawai-type guideblocks can generally deliver 2 to 5 time more pressure than does the DIA for a similar sample volume.  In Kawai-type (6-8) devices eight cubic anvils are used to compress the octahedrally-shaped sample assembly. In the DT25 the two of the eight cubes which sit along the vertical axis can be driven separately from the confining load by individual differential hydraulic pumps. The pumps advance (and retract) hexagonal cross section pistons, atop which sit three tripartite wedges that form a negative cube face that is flush with the first-stage wedges of the Kawai module. The assembly of 8 cubes then is compressed by the first stage wedges, and the top and bottom cubes can be advanced by using the differential pumps to advance the hexagonal pistons, enabling this unique 6-8 multi-anvil apparatus to be used for controlled strain-rate deformation experiments to high strains.

This unique module is a scaled-up and redsigned version of the DT-10, which used 10mm cubes. The DT-25 uses 25mm cubes which allows us to apply higher loads to the system. It is housed in a 1100 ton press with two 250 ton differential jacks driving the upper and lower anvils. We have explored several strategies for transparent cubes allowing for full X-ray diffraction pattern and image collection. The current preference is to purchase sintered diamond anvil 25 mm cubes that are commercially available for only about 4 times the cost of tungsten carbide.  We have used such assemblies with 3 mm truncations for several runs at a 500-ton load both in the Stony Brook Sumitomo press and in the 1100 ton press at MAXPD with no critical failures of the sintered diamond anvils to date.

Thanks to the use of the tripartite wedges in the upper and lower pistons, DT25 can serve as a hydrostatic device as well as a deformation device, making it ideal for higher pressure phase equilibrium and equation of state style experiments. Ultrasonic experiments will eventually be possible in this machine, though the first stage wedges and the pistons will need to be milled and machined to allow for the necessary wires to enter and exit the system. Though we have not pushed the module to its limits yet, we anticipate that the DT25 will be the highest pressure multi-anvil uniaxial deformation device in the world, competing with the RDA for maximum achievable pressures.

- MAXPD Abbreviated Development Timeline:

- 03/20/2017 – Beamline controls system approval

- 04/12/2017 – First Light in XPD-D hutch

- 04/12-14/2017 – Comprehensive Radiation Survey of D hutch completed

- 05/22-24/2017 – Technical Commissioning of MAXPD Phase I

- 05/31/2017 – New Partner User Proposal submitted to NSLS-II

- 06/29-07/01/2017 – Technical Commissioning of MAXPD Phase II

- 07/31/2017 – Mounting of motors on equipment stages begins

- 09/26/2017 – Testing of motor controls using Delta Tau native software begins

- 09/27/2017 – Installation of sample imaging camera and YAG scintillator and stages

- 10/02-04/2017 – Technical Commissioning of MAXPD- Phase III

- 11/02/2017 – NSLS-II Safety Approval of main ram hydraulics to 5,000 PSI (550 tons)

- 12/02-04/2017 – FIRST SCIENCE COMMISSIONING EXPERIMENT

- 03/2018 – SCIENCE COMMISSIONING BEAMTIME

- 05/2018 – Full EPICS control of motors and Allen Bradley PLC functions

- 07/2018 – Detector and Imaging IOCs Finalized

- 08/2018 –USER-ASSISTED SCIENCE COMMISSIONING BEAMTIME

- 08/21/2018 – Final Pressure Testing

- 08/31/2018 – Final Endstation Safety Review – Approved!

- 2018-3 Cycle – FULL GENERAL USER PROGRAM @ MAXPD OPENS!

  - Have met all deadlines and milestones presented at COMPRES meeting

- 11/15-28/2018 – General User and Contributing User Beamtime @ MAXPD

- 11/30/2018 – Pressure Sensor Calibrations Under NSLS-II Calibration Control System

- 2019-1 Cycle – Partner User Agreement Goes Into Effect (Hopefully?)

- 02/28/2019 – Mechanical and Logic Drawings of Endstation Installation Under BNL Document Control Vault System

- 02/28/2019 – Final Experimental Safety Review Envelope Revision

- 04/30/2019 – MAXPD Endstation Fully Integrated into XPD ESR Envelope with Documentation and Certification

- MAXPD Development Projections:

We will continue to develop the DT25 apparatus at NSLS-II over the ensuing year.  Our first experiments on this machine were in the last year.  We are working to design cell assemblies for 5mm and 3 mm truncations and pushing to achieve 30 GPa with the latter size cell assembly. So far we are about half way. We also seek to design channels to be cut into the first-stage anvils and wedges used in the DT-25 to allow the proper wiring to be inserted to make ultrasonic experiments possible in this module.

- 6-BM-B Development Highlights and Plans

At the 6BMB beamline, we continue to improve cell assemblies and software for analyzing data.  Recently, we achieved some improvements to the cell design (Li et al, 2018) and Li has developed a new level of software for automated data reduction, including tools for recovering data on samples that were short enough that the echoes overlap.

The planned development activities at 6-BM-B in the 2019-2020 year are centered on necessary hardware and software upgrades. In 2017, we have experienced significant setbacks due to hardware failure as much of our controls systems and computing capabilities are becoming quite aged. In replacing these obsolete components, we expect to see significant gains in performance at the beamline.

We are exploring the possibility of acquiring a new high pressure system to do 3-D tomographic imaging at high pressure.  Our plans are still in early stage and we will need to seek funds to enable this.  However, we are looking at a 1 – 2 year time to accomplish this goal.

| Budget | Budget for | Proposed budget |
| --- | --- | --- |
| Budget Request for 2019-2020 | June 2018-May 2019 | June 2019-May 2020 |
| Salaries |  |  |
| Matthew Whitaker | $88,580.00 | $91,237.40 |
| Haiyan Chen | $92,700.00 | $95,481.00 |
| William Huebsch | $25,000.00 | $25,750.00 |
| Donald Weidner (0/1 mo) | $0.00 | $19,200.00 |
| Total | $206,280.00 | $231,668.40 |
| Fringe Benefits (40% regular, 14% summer) | $84,575.00 | $87,675.36 |
| Total Salary + FB | $290,855.00 | $319,343.76 |
| Travel | $10,000.00 | $10,000.00 |
| Materials and Supplies | $20,000.00 | $39,856.00 |
| Machine shop | $3,580.00 | $4,000.00 |
| Equipment | $0.00 | $6,200.00 |
| Indirect costs (26%MTDC) | $84,353.00 | $97,031.94 |
| Total | $408,788.00 | $476,431.70 |

Current budget (June 2018 – May 2019) and proposed budget (June 2019 – May 2020)

## Budget Justification

The budgets for both the NSLS II (XPD) beamline and the APS (6BMB) beamline are given together.  The salaries include support for Whitaker, Chen, Huebsch, and Weidner.  Whitaker (NSLS II) and Chen (APS) are beamline scientists engaged at the two beam sites.  Huebsch is an electronics engineer, employed part time, who supports all of the electronic issues for these installations.  While employed part time, he virtually works full time and is an expert in dealing with motor drives and electronic controls.  Huebsch is definitely the most valuable technical support person involved in the project. Weidner is the PI and devotes most of his research time to overseeing the program, defining new directions, and organizing community efforts such as the recent workshop. The budget includes a 3% increase in Whitaker’s and Chen’s salaries.  This is the first time that Weidner has been included in the beamline budget.  Here we are requesting one month summer support towards his efforts.  Other personnel that are entirely supported by other Stony Brook resources are Kenneth Baldwin who is engaged about 80% of the time on this project leading the efforts of data acquisition software and motor control software, Samantha Lin, Administrative assistant at about 30% of the time, and Li Li, Research Associate Professor, who generates new experimental protocols and data reduction software, at about 15%.

The 6BMB travel budget for 2019-2020 includes funds for each beamline scientists and one PI, to attend the COMPRES meeting ($800 each), costs for attending one scientific conference for each beamline scientist ($2000 each), and support for 8 trips from New York to Chicago for PI and other staff to support the APS operation ($900 per trip).

Materials and supplies are expected to be used as follows:

| Materials and Supplies | 2019-2020 |
| --- | --- |
| Cell Parts/ Leinenwebber | $7,600.00 |
| 4 computers @ $1200 HP Z240 | $4,800.00 |
| 12 sintered diamond 25 mm anvils @ $1900 | $7,600.00 |
| DIA and DT25 WC anvils | $10,000.00 |
| Other Lab and Electronic Supplies | $10,000.00 |
| Total | $40,000.00 |

The cost for cell parts are based on the last years expenditure for cell parts.  We need new control computers at APS.  We have 2 that have failed and the age of the other 2 average over 10 years.  The system operation requires all of these computers.

We include technical support as provided by the synchrotron facility or the Stony Brook campus.  At the beamline, we require riggers to move heavy objects, machining, or other operations done to the installation.  On campus, we can access machinists at a lower fee.  Riggers at the beamlines often require a minimum number of people for any job.  All together this budget can cover 40 hours at $100/hour.

We are requesting funds for a vertical axis motorized positioning system for the scintillator and beamstop assembly.  This will allow us to reposition this assembly during an experimental run to allow for better optimization of the alignment and image data quality.  This assembly is what allows the camera to view the x-ray image of the sample.  Safety regulations at the NSLS-II requires that no user enter the hutch while the jack pressure is above 50 tons.  Thus, we need to move all items remotely.  $6200 is the quote that we obtained for this object.

## User Satisfaction

End of Experiment Forms for 6-BM-B are included as Appendix 5. MAXPD is currently in our first cycle as a fully-fledged part of the General User Program, with our first day of beamtime being the day this report is due. Therefore, at the present time, we do not have any of these User Satisfaction documents to present.

Appendices

1. A publication list for 2017-2018.

2. A list of all proposals submitted and beamtime usage for 2017-2018 – see MAP_2018_Appendicies.xls

3. The beamtime schedule for the last year’s cycles. – see MAP_2018_Appendicies.xls

4. Workshop report

5. End of run evaluations

6. Users – see MAP_2018_Appendicies.xls

## Appendix 1.  Publications

APS 6BM-B/X17B2 Publication List 2017-2018

2017 Publication List

- Cheung, S. N. C.; Weidner, D. J.; Li, L.; Meredith, P. G.; Chen, H.; Whitaker, M. L.; Chen, X. (2017) Stress distribution during cold compression of a quartz aggregate using synchrotron X-ray diffraction: observed yielding, damage and grain crushing. Journal of Geophysical Research (Solid Earth), 122 (4) 2724-2735

- Whitaker, M.L., Baldwin, K.J., and Huebsch, W.R. (2017) DIASCoPE: Directly Integrated Acoustic System Combined with Pressure Experiments – A new method for fast acoustic velocity measurements at high pressure. Reviews of Scientific Instruments. 88 (3) 034901

- Farla R., Amulele G., Girard J., Miyajima N., Karato SI., (2017) High-pressure and high-temperature deformation experiments on polycrystalline wadsleyite using the rotational Drickamer apparatus (vol 42, pg 541, 2015), Physics and Chemistry of Minerals 44 (3) 235

- L. Tokle, G. Hirth, P. Raterron, N. Dygert, Y. Liang, C. W. Holyoke , “The Pressure and Mg Dependence of Ilmenite and Ilmenite-olivine Aggregates Rheology: Implications for Lunar Cumulate Mantle Overturn”, Lunar Planetary Science Conference Abstracts, April, 2017

- P. Raterron, L. Tokle, N. Hilairet, S. Merkel, G. Hirth, D. Weidner , “Effect of Fe Content on Olive Viscosity and Implications for the Martian Mantle”, Lunar Planetary Science Conference Abstracts, April, 2017

- GN. Zhang, SH. Mei, MS. Song, D. L. Kohlstedt, “Diffusion Creep of Enstatite at High Pressures Under Hydrous Conditions”, Journal of Geophysical Research: Solid Earth, 122, 7718-7728, 2017, doi:10.1002/2017JB014400

- S. Kaboli, P. C. Burnley, G. Xia and H. W. Green II "Pressure dependence of creep in forsterite olivine: comparison of measurements from the D-DIA and Griggs apparatus", Geophysical Research Letters, 44 (21) 10939-10947 2017, DOI: 10.1002/2017GL075177

- S. Kaboli and P. Burnley, "ECCI, EBSD and EPSC Characterization of Rhombohedral Twinning in Polycrystalline α-Alumina Deformed in the D-DIA Apparatus", Journal of Applied Crystallography, 50 (6) 1691-1704, 2017

- S. Kaboli and P. Burnley, “Applications of Electron Channeling Contrast Imaging (ECCI) in Failure Analysis of In-Situ Synchrotron X-Ray Diffraction Deformation Experiments”, Microscopy and Microanalysis , 23 (S1), 568-569, 2017

- MJ Rucks, TD Glotch, ML Whitaker , JB Parise, “Investigation of tissintite formation using in-situ synchrotron X-ray diffraction and multi-anvil techniques”, Meteoritics & Planetary Science, 52 (2017) Page A 298

2018 Publication List

- YT Zou, Y Li, HY Chen, D Welch, YS Zhao, BS Li, “Thermalelasticity and anomalies in the pressure dependence of phonon velocities in niobium”, Applied Physical Letters, 112 011901 2018

- DJ Weidner, L LI, ML Whitaker, R Triplett, “Ultrasonic Acoustic Velocities During Partial Melting of a Mantle Peridotite KLB-1”, Journal of Geophysical Research: Solid Earth, 123 (2018) 1252-1261

- CSN Cheung, DJ Weidner, L LI, PG Meredith, HY Chen, ML Whitaker, XY Chen, “Stress Distribution During Cold Compression of Rocks and Mineral Aggregates Using Synchrotron-based X-Ray Diffraction”, JOVE, 135, 2018, Doi: 10.3791/57555

- A James, MMD Esfahani, WR Woerner,  A Sinclair, L Ehm, AR Oganov, JB Parise, “Theoretical and Experimental Investigations into Novel Oxynitride Discovery in the GaN-TiO2 System at High Pressure”, Crystals, 2 (2018) Article 15, Doi: 10.3390/cryst8020015

- MJ Rucks, ML Whitaker, TD Glotch, JB Parise, “Formation of Tissintite and Its Implications for Impact Studies”, Lunar and Planetary Science Conference, Volume 49 (2018/3)

- F Bejina, M Bystycky, N Terće, ML Whitaker, HY Chen, "Bulk modulus of Fe-rich olivines corrected for non hydrostaticity",  Comptes Rendus Geoscience, https://doi.org/10.1016/j.crte.2018.06.002

- MJ Rucks, ML Whitaker, TD Glotch, JB Parise, SJ Jaret, T Catalano, MD Dyar, “Making Tissintite: Mimicking meteorite in the multi-anvil” American Mineralogist, 103 (2018) 1516-1519. (Editor's Highlight!)

- L Li, ML Whitaker, DJ Weidner, “Note: Elastic wave velocity measurement using ultrasonic system with two-reflectors” Review of Scientific Instruments, 89 (8) (2018) Article # 086105

- L Li, DJ Weidner, ML Whitaker, R Triplett, “Ultrasonic Acoustic Wave Velocities of Neighborite (NaMgF3) Across Orthorhombic to Cubic Phase Boundary at High P-T”, Physics of the Earth and Planetary Interiors, 283 (2018) 38-42

- Kaboli, S. and Burnley P.C., (in press) In-situ X-ray Diffraction Deformation and EPSC Modeling of AZ31 Mg Alloy. Materials Science & Engineering A (Submitted 8/20/2018)

- Burnley P. C. and Kaboli S. (in press) Elastic Plastic Self Consistent (EPSC) Modeling of San Carlos Olivine Deformed in a D-DIA Apparatus.  American Mineralogist (submitted 6/1/2018)

Manuscripts Submitted

- Mohiuddin a., Karato S-I., Girard J., “Simulating deep slab deformation by synchrotron in-situ high-pressure experiment”, Nature Geosciences (submitted)

- Hansen, L.N.; Thom, C.A.; Kumamoto, K.M.; Wallis, D.; Durham, W.B.; Goldsby, D.L.; Breithaupt, T.; and Kohlstedt, D.L “Low-temperature plasticity in olivine: Grain size, strain hardening, and the strength of the lithosphere”. Journal of Geophysical Research: Solid Earth. (submitted)

PhD Theses

- Dharmagunawardhane, HAN (2017) Synthesis of oxynitride materials for solar water splitting: investigations with ambient pressure and high pressure synthesis techniques, Ph. D. Thesis, Stony Brook University

- Chen, T (2017) Elasticity of Coesite and Stishovite: Implications for the Earth's Mantle, Ph. D. Thesis, Stony Brook University

- Kumamoto, K (2018)  Exploring the rheological properties of the upper mantle : from the field to the laboratory, Ph. D. Thesis, Stanford Univeristy

Masters Theses

- Millard, Joseph (2018)  “Pressure Dependence Of The Strength Of Magnesite Deforming By Low Temperature Plasticity, Diffusion Creep, Or Dislocation Creep”, University of Akron, http://rave.ohiolink.edu/etdc/view?acc_num=akron1526913343559104

- McDaniel, Caleb (2018) “Effect of grain size on the strength of magnesite deforming by diffusion creep or low temperature plasticity mechanisms.” University of Akron, http://rave.ohiolink.edu/etdc/view?acc_num=akron1526571269872349

## Appendix 4.  Workshop Report

- Report on COMPRES Workshop: Envisioning the Next Generation of In-situ Synchrotron X-ray Techniques in Large-Volume High Pressure Apparatus for Mineral and Rock Physics

On September 28-30, 2018 a COMPRES sponsored workshop entitled “Envisioning the Next Generation of In-situ Synchrotron X-ray Techniques in Large-Volume High Pressure Apparatus for Mineral and Rock Physics” was held at the Advanced Photon Source, Argonne National Lab. The goal of the workshop was to explore how cutting-edge synchrotron techniques, including those that can characterize 3D distributions of properties and micromechanical response such as phase, density, crystallographic orientation or stress states, can be integrated into large volume apparatus experiments. Workshop participants learned about the current state of the art in synchrotron-based rock deformation experiments and in-situ tomographic imaging of features with contrasting density such as fractures and melts. Participants also learned about the current state of the art in 3D mapping of grain morphology, orientation, and stress states via high energy diffraction microscopy (HEDM), the 3D mapping of phase distribution using diffraction tomography, absorption computed tomography and energy dispersive x-ray diffraction. These techniques are currently being utilized to characterize the distribution and evolution of phases in materials from bones to batteries and the 3D distribution of stresses in metals as they are being deformed. Speakers from a broad spectrum of rock physics and deformation specializations talked about problems in their research areas that could potentially be solved with in-situ observations. Brain storming sessions identified first experiments, new collaborations and future directions.

The workshop spanned 2½ days.  The agenda is given in Appendix 4a.  59 attendees came from rock physics, mineral physics, and synchrotron physics (Appendix 4b). Scientists from APS, NSLS II, CHESS, Solei, and Petra III represented synchrotron advances from their facility.

One of the results of the workshop was the decision to seek funding for an NSF Research Coordination Network grant that would facilitate continued dialog, future small and large workshops and additional efforts to push this new area of science forward. Wenlu Zhu was chosen to head up the new RCN effort. The full agenda as well as talks presented at the workshop are available at http://www.largevolumercn.net.

The workshop was organized by Pamela Burnley, Haiyan Chen, William Durham, Shun Karato, Andreas Kronenberg, Donald Weidner, Matthew Whitaker, and Wenlu Zhu. APS site hosting was provided by Dr. Jonathan Almer, Ms. Linda Carlson with assistance from Dr. Yue Meng and Dr. Ke Yuan. Additional logistical support was provided by Mr. Richard Fenner, Mr. Barry Switzer, and Ms. Amy Mikuta from APS and Ms. Samantha Lin from Stony Brook University. Evening refreshments were provided by Rockland Research Corp.

Appendix 4a:  Workshop Agenda

Appendix 4b: Attendees

| Name | Email | Institution |
| --- | --- | --- |
| Andreas Kronenberg | kronenberg@geo.tamu.edu | Texas A&M University |
| Andrew Cross | andcross@sas.upenn.edu | University of Pennsylvania |
| Andy Campbell | campbell@geosci.uchicago.edu | University of Chicago |
| Anwar Mohiuddin | anwar.mohiuddin@yale.edu | Yale University |
| Blake Sturtevant | bsturtev@lanl.gov | Los Alamos National Laboratory |
| Caleb Holyoke | cholyoke@uakron.edu | University of Akron |
| Carl Agee | agee@unm.edu | COMPRES/UNM |
| Caroline Bollinger | <c.bollinger@wustl.edu> | Washington University in St. Louis |
| Changyong Park | cypark@anl.gov | Argonne National Laboratory |
| Christopher Thom | chrthom@sas.upenn.edu | University of Pennsylvania |
| Darren Pagan, Ph.D. | dcp99@cornell.edu | Cornell High Energy Synchrotron Source |
| David Kohlstedt | dlkohl@umn.edu | University of Minnesota |
| David L. Goldsby | dgoldsby@sas.upenn.edu | University of Pennsylvania |
| Donald Weidner | dweidner@stonybrook.edu | Stony Brook University |
| Douglas Schmitt | schmitt@purdue.edu | EAPS, Purdue |
| Eglantine Boulard | eglantine.boulard@upmc.fr | IMPMC, Sorbonne France - imaging partial melts |
| eric cao | cao.y.1@duracell.com | duracell |
| Eric Dooryhee | dooryhee@bnl.gov | Brookhaven National Laboratory |
| Guijie Sang | gbs5103@psu.edu | The Penn State University, University Park, Pa |
| Haiyan Chen | haiyan@bnl.gov | Stony Brook University |
| Hans J. Mueller | hans-joachim.mueller@partner.kit.edu | KIT Karlsruhe Institute of Technology |
| Hector M. Lamadrid | lamadrid56@gmail.com | University of Missouri |
| Helene Couvy | helene.couvy@wustl.edu | Washington University in St Louis |
| Hemant Sharma | hsharma@aps.anl.gov | Argonne National Laboratory |
| HYUNCHAE CYNN | cynn1@llnl.gov | Lawrence Livermore National Laboratory |
| Jennifer Girard | jennifer.girard@yale.edu | Yale university |
| Jianhua Gong | gongjh@mit.edu | MIT/WHOI Joint Program |
| Jihui Ding | dingjihui@tamu.edu | Texas A&M University, Department of Geology and Geophysics |
| Jin Liu | jinliuyc@gmail.com | Stanford University, USA |
| Jiuhua Chen | Chenj@fiu.edu | Florida International University |
| John Okasinski | okasinski@aps.anl.gov | Argonne National Laboratory |
| Jonathan D. Almer | almer@aps.anl.gov | Argonne National Laboratory |
| Ke Yuan | kyuan@anl.gov | Argonne National Laboratory |
| Leif Tokle | Leif_Tokle@brown.edu | Brown University |
| Lisa Danielson | lisa.r.danielson@nasa.gov | Jacobs, JETS - NASA JSC |
| Mark Rivers | rivers@cars.uchicago.edu | University of Chicago |
| Matthew L. Whitaker | matthew.whitaker@stonybrook.edu | Stony Brook University |
| Pamela Burnley | Pamela.Burnley@unlv.edu | UNLV |
| Phil Skemer | pskemer@wustl.edu | Washington University in St. Louis |
| Rellie Goddard | catherine.goddard@univ.ox.ac.uk | University of Oxford |
| Rick Rowland II | Richard.L.Rowland@NASA.Gov | Jacobs JETS at NASA Johnson Space Center Astromaterials Research and Exploration Science |
| Robert Farla | robert.farla@desy.de | Deutsches Elektronen-Synchrotron (DESY) |
| - Ross Hrubiak | hrubiak@anl.gov | Argonne National Laboratory |
| Rui Zhang | zhangrui117@gmail.com | Penn State University |
| Shun-ichiro Karato | shun-ichiro.karato@yale.edu | Yale University |
| Taryn Traylor | traylt1@unlv.nevada.edu | UNLV |
| Terry E Tullis | terry_tullis@brown.edu | Brown University |
| Tim Officer | officer@cars.uchicago.edu | GSECARS |
| Ulrich Faul | hufaul@mit.edu> | Massachusetts Institute of Technology |
| vitali prakapenka | prakapenka@cars.uchicago.edu | University of Chicago |
| Wenlu Zhu | wzhu@umd.edu | University of Maryland |
| William Durham | wbdurham@mit.edu | Massachusetts Institute of Technology |
| Xianghui Xiao | xianghuix@gmail.com | NSLS II |
| Yanbin Wang | wang@cars.uchicago.edu | APS/ GSECARS |
| Yu Shu | yushu@anl.gov | HPCAT of XSD at APS |
| Yue Meng | ymenng@anl.gov | XSD, Advanced Photon Source |
| Yuming Xiao | poexiao@anl.gov | HPCAT, XSD, Argonne National Lab |
| Zhong Zhong | zhong@bnl.gov | NSLS II |
| Ziyan Li | liziyan1992@gmail.com | Penn State |

Appendix III: Images

Group picture.

Sunday, before break up, discussion of future plans.

## Apppendix 5.  End of Run responses

APS END OF EXPERIMENT FORM

EEF Created date: 04/01/2018

Beamline: 6-BM-A,B Experiment ID: 126221

Exp. Start Date: 15-MAR-18 Exp. End Date: 19-MAR-18

Spokesperson: Hansen, Lars Experiment Host: CHEN, HAIYAN

Form Filled Out By: Hansen, Lars Institution: University of Oxford

Experiment Title: An experimental study of size effects in olivine deforming via low-temperature plasticity

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 10/31/2017

Beamline: 6-BM-A,B Experiment ID: 121019

Exp. Start Date: 27-OCT-17 Exp. End Date:

Spokesperson: Gwanmesia, Gabriel Experiment Host: CHEN, HAIYAN

Form Filled Out By: Gwanmesia, Gabriel Institution: Delaware State University

Experiment Title: Sound Wave Velocities and Elasticity of Hydrated Mantle Minerals at High Pressures and High

Temperatures.

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:2

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

User Comments: There were some technical issues at the beginning that caused us to loss half of our beamtime

schedule, but once resolved, the beamline staff were quite supportive and gave in much time

that enable the acquisition of a very comprehensive set of data. In general, we are happy with

the beamline data acquisition system and the assistance of the staff.

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 11/09/2017

Beamline: 6-BM-A,B Experiment ID: 121227

Exp. Start Date: 01-NOV-17 Exp. End Date: 03-NOV-17

Spokesperson: Burnley, Pamela Experiment Host: ALMER, JON

Form Filled Out By: Burnley, Pamela Institution: University of Nevada, Las Vegas

Experiment Title: Examination of low strain behavior of olivine polycrystals as a function of pressure using the D-DIA

apparatus

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:2

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

User Comments: We had some network problems with the camera control software (it kept dropping the

connection) the beamline staff showed us how to deal with the problem. Everything else was

fine.

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 11/07/2017

Beamline: 6-BM-A,B Experiment ID: 121393

Exp. Start Date: 03-NOV-17 Exp. End Date: 05-NOV-17

Spokesperson: Triplett, Richard Experiment Host: CHEN, HAIYAN

Form Filled Out By: Triplett, Richard Institution: State University of New York, Stony Brook

Experiment Title: Elasticity of nonrecoverable perovskite minerals at mid-mantle conditions

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:3

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/01/2018

Beamline: 6-BM-A,B Experiment ID: 121555

Exp. Start Date: 07-NOV-17 Exp. End Date: 10-NOV-17

Spokesperson: Sims, Melissa Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Investigation of Jadeite Behavior at High Temperature and Pressure

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:2

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:2

Data analysis software at the beamline:2

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 11/13/2017

Beamline: 6-BM-A,B Experiment ID: 121719

Exp. Start Date: 09-NOV-17 Exp. End Date:

Spokesperson: Wang, Pei Experiment Host: CHEN, HAIYAN

Form Filled Out By: Wang, Pei Institution: University of Nevada, Las Vegas

Experiment Title: Flow Mechanism and Constitutive Law of Nano- & Micron-grained Superhard Materials: In-situ

Deformation Study at Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 11/20/2017

Beamline: 6-BM-A,B Experiment ID: 121935

Exp. Start Date: 14-NOV-17 Exp. End Date: 16-NOV-17

Spokesperson: Wang, Liping Experiment Host: CHEN, HAIYAN

Form Filled Out By: Wang, Liping Institution: University of Nevada, Las Vegas

Experiment Title: Effect of Oxygen Fugacity on the Thermal and Elastic Properties of Ferric-Iron-Bearing Ahrensite

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 11/21/2017

Beamline: 6-BM-A,B Experiment ID: 122052

Exp. Start Date: 15-NOV-17 Exp. End Date: 18-NOV-17

Spokesperson: Tokle, Leif Experiment Host: CHEN, HAIYAN

Form Filled Out By: Tokle, Leif Institution: Brown University

Experiment Title: The effects of ilmenite on lunar mantle rheology

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 12/19/2017

Beamline: 6-BM-A,B Experiment ID: 122192

Exp. Start Date: 18-NOV-17 Exp. End Date: 22-NOV-17

Spokesperson: Holyoke, Caleb Experiment Host: CHEN, HAIYAN

Form Filled Out By: Holyoke, Caleb Institution: University of Akron

Experiment Title: Magnesite Deformation and Potential Roles in the Slip and Seismicity of Subduction Zones

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

User Comments: Haiyan Chen was very helpful.

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 12/18/2017

Beamline: 6-BM-A,B Experiment ID: 122440

Exp. Start Date: 24-NOV-17 Exp. End Date: 26-NOV-17

Spokesperson: Cheung, See Nga Cecilia Experiment Host: CHEN, HAIYAN

Form Filled Out By: Cheung, See Nga Cecilia Institution: University of Wisconsin-Madison

Experiment Title: Stress distribution during cold compression of siltstone

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 12/01/2017

Beamline: 6-BM-A,B Experiment ID: 122486

Exp. Start Date: 26-NOV-17 Exp. End Date: 29-NOV-17

Spokesperson: Hansen, Lars Experiment Host: CHEN, HAIYAN

Form Filled Out By: Hansen, Lars Institution: University of Oxford

Experiment Title: An experimental study of size effects in olivine deforming via low-temperature plasticity

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:3

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 02/16/2018

Beamline: 6-BM-A,B Experiment ID: 122765

Exp. Start Date: 29-NOV-17 Exp. End Date: 04-DEC-17

Spokesperson: Karato, Shun-ichiro Experiment Host: CHEN, HAIYAN

Form Filled Out By: Girard, Jennifer Institution: Yale University

Experiment Title: In-situ rheological study of MgO at lower mantle conditions, using the Rotational Drickamer apparatus

coupled with X-ray synchrotron beam.

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 02/16/2018

Beamline: 6-BM-A,B Experiment ID: 122960

Exp. Start Date: 04-DEC-17 Exp. End Date: 06-DEC-17

Spokesperson: Karato, Shun-ichiro Experiment Host: CHEN, HAIYAN

Form Filled Out By: Girard, Jennifer Institution: Yale University

Experiment Title: Deformation of olivine during phase transformation to wadsleyite

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/01/2018

Beamline: 6-BM-A,B Experiment ID: 126777

Exp. Start Date: 27-MAR-18 Exp. End Date: 28-MAR-18

Spokesperson: Whitaker, Matthew Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Reexamining in situ Pressure Markers Using Ultrasonics: A Standardless Approach

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

User Comments: That guy Matt is pretty awesome.

Beam wandered during the experiment. Takeoff angle issue from source?

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/01/2018

Beamline: 6-BM-A,B Experiment ID: 126863

Exp. Start Date: 28-MAR-18 Exp. End Date: 30-MAR-18

Spokesperson: Rucks, Melinda Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Ca-rich plagioclase at extreme conditions: An in situ case study of the formation of tissintite

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:2

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

User Comments: Beam moved quite a bit during the run.

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/01/2018

Beamline: 6-BM-A,B Experiment ID: 126968

Exp. Start Date: 30-MAR-18 Exp. End Date: 01-APR-18

Spokesperson: Whitaker, Matthew Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Expanding the Available Pressure Range Attainable in Acoustic Velocity Studies at 6-BM-B

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

User Comments: Seriously, that guy Matt is really good.

X-ray beam wandering that was present in previous runs did not occur here. Whatever fixed it,

thanks!

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/07/2018

Beamline: 6-BM-A,B Experiment ID: 127023

Exp. Start Date: 01-APR-18 Exp. End Date: 05-APR-18

Spokesperson: Triplett, Richard Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Elasticity of nonrecoverable perovskite minerals at mid-mantle conditions

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:2

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/07/2018

Beamline: 6-BM-A,B Experiment ID: 127403

Exp. Start Date: 05-APR-18 Exp. End Date:

Spokesperson: Weidner, Donald Experiment Host: CHEN, HAIYAN

Form Filled Out By: Whitaker, Matthew Institution: State University of New York, Stony Brook

Experiment Title: Effect of coexisting high and low pressure phases on elastic properties at MHz frequencies

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :2

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:2

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:2

Detectors:2

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/08/2018

Beamline: 6-BM-A,B Experiment ID: 127585

Exp. Start Date: 07-APR-18 Exp. End Date:

Spokesperson: Gwanmesia, Gabriel Experiment Host: CHEN, HAIYAN

Form Filled Out By: Triplett, Richard Institution: Delaware State University

Experiment Title: Sound Wave Velocities and Elasticity of Hydrated Mantle Minerals at High Pressures and High

Temperatures.

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:2

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/17/2018

Beamline: 6-BM-A,B Experiment ID: 127963

Exp. Start Date: 12-APR-18 Exp. End Date:

Spokesperson: Wang, Liping Experiment Host: CHEN, HAIYAN

Form Filled Out By: Wang, Wendan Institution: University of Nevada, Las Vegas

Experiment Title: Crystal Structures and Phase Transitions in SnSe at Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/17/2018

Beamline: 6-BM-A,B Experiment ID: 127965

Exp. Start Date: 12-APR-18 Exp. End Date: 14-APR-18

Spokesperson: Wang, Liping Experiment Host: CHEN, HAIYAN

Form Filled Out By: Wang, Wendan Institution: University of Nevada, Las Vegas

Experiment Title: Effect of Oxygen Fugacity on the Thermal and Elastic Properties of Ferric-Iron-Bearing Ahrensite

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 04/17/2018

Beamline: 6-BM-A,B Experiment ID: 128103

Exp. Start Date: 14-APR-18 Exp. End Date:

Spokesperson: Wang, Wendan Experiment Host: CHEN, HAIYAN

Form Filled Out By: Wang, Wendan Institution: University of Nevada, Las Vegas

Experiment Title: Phase diagram of PbTeO3 under Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 06/16/2018

Beamline: 6-BM-A,B Experiment ID: 129962

Exp. Start Date: 12-JUN-18 Exp. End Date:

Spokesperson: Wang, Pei Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Wang, Pei Institution: University of Nevada, Las Vegas

Experiment Title: Flow Mechanism and Constitutive Law of Nano- & Micron-grained Superhard Materials: In-situ

Deformation Study at Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 06/18/2018

Beamline: 6-BM-A,B Experiment ID: 130150

Exp. Start Date: 16-JUN-18 Exp. End Date:

Spokesperson: Wang, Pei Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Wang, Pei Institution: University of Nevada, Las Vegas

Experiment Title: Deformation Mechanism of Nanocrystalline & Microncrystalline Hard and Superhard Materials: In-situ

Study at Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 06/22/2018

Beamline: 6-BM-A,B Experiment ID: 130227

Exp. Start Date: 19-JUN-18 Exp. End Date: 21-JUN-18

Spokesperson: Wang, Wendan Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Wang, Pei Institution: University of Nevada, Las Vegas

Experiment Title: Phase transition of PbTeO3 and Pb2Te3O8 under Simultaneous High Pressure and Temperature

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 07/08/2018

Beamline: 6-BM-A,B Experiment ID: 130881

Exp. Start Date: 06-JUL-18 Exp. End Date: 08-JUL-18

Spokesperson: Triplett, Richard Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Triplett, Richard Institution: State University of New York, Stony Brook

Experiment Title: Elasticity of nonrecoverable perovskite minerals at mid-mantle conditions RAP

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :n/a

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:n/a

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 07/15/2018

Beamline: 6-BM-A,B Experiment ID: 131060

Exp. Start Date: 11-JUL-18 Exp. End Date:

Spokesperson: Gwanmesia, Gabriel Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Gwanmesia, Gabriel Institution: Delaware State University

Experiment Title: Sound Wave Velocities and Elasticity of Hydrated Mantle Minerals at High Pressures and High

Temperatures.

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:n/a

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:n/a

User Comments: We lost 2 days of beam time due vacuum problem and thus could not complete our

experiments.

1/1

APS END OF EXPERIMENT FORM

EEF Created date: 07/30/2018

Beamline: 6-BM-A,B Experiment ID: 131241

Exp. Start Date: 16-JUL-18 Exp. End Date: 23-JUL-18

Spokesperson: Karato, Shun-ichiro Experiment Host: OKASINSKI, JOHN S.

Form Filled Out By: Girard, Jennifer Institution: Yale University

Experiment Title: Deformation of olivine during phase transformation to wadsleyite

Question 1: Please rate your satisfaction with the following aspects of this experiment. (mandatory)

Add comments for any especially positive or negative experience. If the item doesn't apply to you, choose n/a.

1-Highly Satisfied; 2-Satisfied; 3-Neither satisfied nor unsatisfied;4-Unsatistifed;5-Very unsatisfied;

Available choices and answer(s): Pre-arrival support from the User Office :1

Pre-arrival shipping and handling:1

Support from the beamline staff:1

X-ray optics at the beamline:1

Control and data-acquisition software at the beamline:1

Data analysis software at the beamline:1

Detectors:1

Post-experiment arrangements for shipping and handling:1

1/1