Partnership for Extreme Crystallography (PX^2)

2017 COMPRES Annual Report

November 2016 – October 2017

Prepared by Dongzhou Zhang, Przemyslaw Dera, University of Hawaii at Manoa

Overview

Partnership for eXtreme Xtallography (PX^2) is providing new capabilities for high-pressure diamond anvil cell research at the GSECARS APS beamline.  The PX^2 project is a collaboration between the University of Hawaii and GSECARS, supported by COMPRES and hosted by GSECARS at experimental station 13-BM-C.  This beamline provides focused X-rays at two fixed energies: 15 and 28.6 keV and a unique 6-circle heavy duty diffractometer, optimized for a variety of advanced crystallography experiments including interface studies, powder and single crystal structure determination, equation of state studies and thermal diffuse scattering.  Currently we support high pressure and temperature experiments using resistively heated diamond anvil cells, and have achieved P-T conditions of 100 GPa and 1500 K. A new compact optical platform has been installed and provides ruby-fluorescence pressure determination, black-body radiation temperature measurement and Raman spectroscopy capabilities.

The beamline has now a well-established and very active user base. We have been receiving a steady stream of GUP proposals that each cycle well exceed the 50% fraction of beam time for which PX^2 was planned. Since the last annual report the number of published peer-reviewed papers resulting from the PX^2 user program has reached 15, with 5 papers published in premiere journals with impact factor higher than 9.

In 2017 the main technical development thrust was placed on the laser heating system. The 200W NIR fiber laser has been connected with the PX^2 optical platform and fully integrated with the experimental station safety and interlock systems. A series of extensive performance and calibration tests was conducted on single crystals at ambient pressure (summer 2017), and powders in diamond anvil cell (fall 2017). A report on these developments was presented at the COMPRES Annual Meeting by Dera. Further details are included in the sections below. We plan to make the laser heating system available to PX^2 users by the end of 2018.

In anticipation of the capital equipment acquisition, a new PHOTON II area detector, which was included in the COMPRES renewal proposal budget, we established a close collaboration with the detector vendor, Bruker AXS to develop the necessary hardware and software solutions that will be needed, when this innovative device becomes available at PX^2. With a generous loan from Bruker, we have been using one of the PHOTON II devices on loan, since April 2017. A mechanical mounting system has been designed and manufactured to attach the detector to the Newport 6-circle diffractometer. An extensive software development effort by Dr. M. Rivers produced an EPICS driver for the new type of detector. With the new EPICS driver, the data collection software can integrate control of the detector with the rotation of the sample. A series of preliminary performance tests has also been conducted.

In 2017 we added to the PX^2 system a new radiography/tomography camera, which has been successfully used for studying solid inclusions in diamonds (in collaboration with Northwestern Univ. group).

These new capabilities are available to all researchers interested in studying deep earth materials through the APS General User Proposal system.

Response to specific recommendation from the COMPRES feedback to previous annual report:

- Relationship with ALS 12.2.2

- Better coordination with 12.2.2 to avoid duplication of effort

The PX^2 team initiated more direct interaction between the two facilities and their personnel. Dr. Beavers was invited to speak as lecturer at the workshop on multigrain diffraction at high pressure, co-organized by Dera in November 2016 at Argonne. The PX^2 team (Zhang and Dera) initiated and hosted a teleconference with the ALS 12.2.2. team (Williams and Beavers), which took place on November 8, 2017. During the teleconference possible avenues of future collaboration were discussed. Both teams agreed to hold similar teleconferences on a regular base, once every synchrotron scheduling period.

- Users should be able to process data from either facility with the same set of software

Since 2016 PX^2 offers the commercial software package, Bruker APEX 3, as one of the data analysis options for single crystal diffraction images collected at 13BM-C with MAR 165CCD detector. The same software is available to process data from ALS 12.2.2. Training materials explaining the data analysis procedures have been made available to PX^2 users, and a training session (taught by Dr. Michael Ruf from Bruker AXS), was offered at the multigrain diffraction workshop at APS in November 2016.

- Facility should organize a hands-on workshop for user training

In addition to the multigrain workshop in 2016, in late July 2018 we are planning to host a major international high pressure meeting – a workshop of the International Union of Crystallography Commission on High Pressure in Honolulu, HI. The main conference event will be proceeded by a one day training event for students and postdocs focused on experimental procedures and software for single crystal diffraction. Scientists from both PX^2 and ALS 12.2.2. will participate as instructors. An EO&ID proposal to support the cost of this event and enable participation of COMPRES community students will be submitted separately.

- Status update on laser heating system

Please, see the description in the following sections

Proposed new model for COMPRES personnel at GSECARS

Currently, two COMPRES facilities projects are active at GSECARS, the Gas Loading Facility (GLF) and PX^2. GLF operates with one staff person funded at 50% FTE by COMPRES and 50% by GSECARS. Besides the GLF operation, his current main responsibilities include support of GSECARS user program for in situ Brillouin spectroscopy at the experimental station 13BM-D.

PX^2 was originally planned for operation involving usage of up to 50% of beam time in experimental station 13BM-C and one staff Beamline Scientist. At the start of the project, during the commissioning phase, in addition to the Beamline Scientist, a second Ph.D. level staff person, the COMPTECH Technology Officer, was available to help with the instrument design and testing. After her departure, this role has been played by UH graduate student intern, Yi Hu. The PX^2 program has been open for general user access for almost 3 years, since the beginning of 2015.

The PX^2 program shares the same 13BM-C experimental station with GSECARS surface scattering program. Surface scattering has a fairly small user community and does not receive too many beamtime proposals, but is scientifically valuable. GSECARS has been dividing the time in 13BM-C between PX^2 and surface scattering based on proposal pressure. PX^2 has been experiencing steadily increasing demand for beam time, and currently the COMPRES share of beam time at 13BM-C is about 80%. DAC experiments are running continuously for about 10 weeks at a time.

APS operates in 24h/day, 6 days per week mode. The Beamline Scientist is present on site every day (including weekends) during the regular hours 9am-5pm, and on call at all times, including nights, to help with troubleshooting of any technical difficulties. At APS the individual beam time cycles are 3 months long, separated by a month of ring maintenance and machine studies. It is very difficult and exhausting for a single beamline scientist to support users every day for several months at a time. For full time operation, typically this task is handled by a team of two Beamline Scientists, who can take turns supporting users, and take few days of rest/relief. Unfortunately, GSECARS does not currently have enough staff to delegate another person to help with PX^2 user program on regular basis.  For comparison, the ALS 12.2.2 facility has 3 full time beamline scientists plus a technician to operate a full-time program with one experimental station operating at one time. The COMPRES program at that facility accounts for 50% of beam time, and is supported by two full-time COMPRES-funded beamline scientists.

After a series of discussions with the GSECARS team, we would like to propose a solution to the PX^2 personnel challenge. We request that the COMPRES support for the GLF position is increased from 50% to 100%, and that the scope of responsibilities for this position is modified to include only COMPRES-funded projects, namely GLF activities, and user support at 13BM-C/PX^2. GSECARS will assign the user support responsibilities for 13BM-D Brillouin spectroscopy program to another staff person, fully funded from GSECARS budget. The employment of the new full time Beamline Scientist/GLF operator would be through the GSECARS GLF subcontract.

Scientific highlights

15 peer-reviewed papers have been published between 2016 and 2017 from the work done at the PX^2 facility. The full publication list is attached in Appendix II. Following are selected research highlights.

- Isosymmetric pressure-induced bonding increase changes compression behavior of clinopyroxenes across jadeite-aegirine solid solution in subduction zones (Xu et al., JGR-Solid Earth, 122, B13502.). Synchrotron-based single-crystal X-ray diffraction experiments were carried out on a natural aegirine [NaFe3+Si2O6] sample at ambient temperature and pressures to 60 GPa, simulating conditions within the coldest part of a subduction zone consisting of old lithosphere. The diffraction data reveal no obvious sign of structural phase transition in aegirine within this pressure range, however several relevant structural parameter trends change noticeably at approximately 24 GPa, indicating presence of the previously predicted isosymmetric bonding change, related to increase of coordination number of Na+ at M2 site. The pressure-volume data, fit with 3rd order Birch-Murnaghan(BM3) equation of state over the whole pressure range, yields KT0 = 126(2) GPa and K′T0 = 3.3(1), while separate BM3 fits performed for the 0-24.0GPa, and 29.9-60.4 GPa pressure ranges give KT0=118(3), K’T0= 4.2(3) and KT0 = 133(2), K′T0=3.0(1), suggesting that the structure stiffens as a result of the new bond formation. Aegirine exhibits strong anisotropic compression with unit strain axial ratios ε1:ε2:ε3=1.00:2.44:1.64. Structural refinements reveal that NaO8 polyhedron is the most compressible, and SiO4 tetrahedron has the lowest compressibility. The consequence of bonding transition is that the compressional behavior of aegirine below ~24 GPa and above that pressure is quite different, with likely consequences for relevant thermodynamic parameters and ion diffusion coefficients.

- Metastable augite at high pressure and temperature conditions and its implications for subduction zone dynamics (Xu et al., Am. Min, 102(7), 1516-1524). We present the synchrotron-based single-crystal X-ray diffraction study on a natural diopside-dominated augite at high pressures and temperatures to ~27 GPa and 700 K to simulate conditions within the cold subducting slabs. Our results indicate that augite is stable over this pressure and temperature range. The third order high-temperature Birch–Murnaghan equation was used to fit the pressure-volume-temperature data and yields the thermoelastic parameters. The strain analysis is conducted and the results show that the compression along the three principal stress directions is highly anisotropic with ε1:ε2:ε3=1.98:2.43:1.00. Additionally, high-pressure structural refinements of polyhedral geometry, bond lengths and O3-O3-O3 angle were investigated to ~27 GPa at ambient temperature. Pressure dependences of polyhedral volumes and distortion indicate that the incorporation of Al3+ in the T site would significantly changes the compression behavior of the T-tetrahedron in augite.

- Dehydrogenation of goethite in Earth’s deep lower mantle (Hu et al., PNAS, 114(7), 1498-1501). We found at high pressure–temperature (P-T) that the goethite FeO2H transforms to P-phase FeO2 via a two-step dehydrogenation process. First it releases some hydrogen to form P-phase FeO2Hx, and then it continuously releases the remaining hydrogen through prolonged heating. Using a combination of theoretical simulations and high-pressure–temperature experiments, we calibrated the x dependence of molar volume of the P phase. Within the current range of experimental conditions, we observed a compositional range of P phase of 0.39 < x < 0.81, corresponding to 19–61% dehydrogenation. This work provides an important example that the dehydration reaction changes to dehydrogenation of FeO2H at the lower mantle conditions and the cycles of hydrogen and water become separated.

- High pressure equation of state of magnesiowüstite (Finkelstein et al., Am. Min, 102 (8), 1709-1717). Magnesiowüstite, (Mg,Fe)O, solid solutions with high iron enrichment may be an important component of ultra-low velocity zones at Earth’s core-mantle boundary. However, to date there have been few high-precision studies on the elastic properties of these materials. In this study we present results on the compression of (Mg0.22Fe0.78)O (Mw78) in both neon and helium pressure media using single-crystal diffraction to ~55 GPa. In addition, our sample was characterized by time domain synchrotron Mössbauer spectroscopy at ambient pressure using an extended time range that resulted in vastly improved energy resolution. We make two main observations: K0T decreases as a function of increasing iron content and K'0T remains at approximately 4.0 independent of iron content. In combination with previously reported thermal parameters, we extrapolate elastic parameters of Mw78 to deep-Earth conditions and determine estimated density profiles.

- Comparing the pressure-induced structural behavior of CuCr2O4 and CuCr2Se4 spinels (Efthimiopoulos et al., JPC-C 121(30), 16513-16520). We have conducted high-pressure measurements on the CuCr2O4 and CuCr2Se4 spinels, in order to unravel the structural systematics of these materials under compression. Our studies have revealed diverse structural behavior in these two compounds. In particular, CuCr2O4 retains its ambient-pressure I41/amd structure up to 50 GPa. Close inspection of the lattice and interatomic parameters reveals a compressibility change near 23 GPa, which is accompanied by an expansion of the apical Cr-O bond distances. We speculate that an outer Cr3+ 3d orbital reorientation might be at play in this system, manifesting as the change in compressibility at that pressure point. On the other hand, CuCr2Se4 undergoes a structural transformation from the starting Fdm phase towards a monoclinic structure initiating at ~ 8 GPa and completed at ~20 GPa. This high-pressure behavior resembles that of ZnCr2Se4, and it appears that, unlike similar chalcogenide Cr-spinels, steric effects take a leading role in this pressure-induced Fdm→monoclinic transition. Close comparison of our results with the reported literature yields significant insights behind the pressure-induced structural systematics of this important family of materials, thus allowing for both the careful manipulation of the structural/physical properties of these systems by strain, as well as promoting our understanding of similar pressure-induced effects in relevant systems.

Beamline Personnel

Project PI: Przemyslaw Dera (University of Hawaii at Manoa)

Przemek is the PI of the PX^2 project.

Beamline scientist: Dongzhou Zhang (University of Hawaii at Manoa)

Dongzhou is the beamline scientist of the PX^2 project. He is responsible for the user support of PX^2. Dongzhou has been working in PX^2 since Jan 2015, and is fully supported by COMPRES.

Related personnel:

Peter Eng (GSECARS, University of Chicago)

Peter is the beamline scientist of GSECARS in charge of the surface diffraction program of 13BMC. He is involved in the beamline operations and beamline upgrade of the PX^2 program.

Joanne Stubbs (GSECARS, University of Chicago)

Joanne is the beamline scientist of GSECARS in charge of the surface diffraction program of 13BMC. She is involved in the beamline operations and beamline upgrade of the PX^2 program.

Vitali Prakapenka (GSECARS, University of Chicago)

Vitali is the beamline scientist of GSECARS in charge of the diamond anvil cell program. He is involved in the beamline operations and beamline upgrade of the PX^2 program

Mark Rivers (GSECARS, University of Chicago)

Mark is the co-director of GSECARS. He is involved in the beamline upgrade of the PX^2 program.

Experimental instruments

- 50% beamtime, sharing with surface diffraction program of GSECARS

- X-ray energy of 15/28.6 keV, focused to 15×15 micron2

- Monochromized X-ray with eV bandwidth

- 6-circle kappa geometry diffractometer

- Area detector (MAR165 CCD, Pilatus 100K, Bruker Photon II)

- Resistive heating setup, including power supply, thermal couple temperature reading and water cooling

- Laser heating setup with black-body radiation temperature reading

- Compact optical platform capable of doing ruby-fluorescence pressure determination and Raman spectroscopy measurements

List of users and affiliations, 2016-2017

Yi Hu, Hannah Shelton, Jing Gao, Jingui Xu, Xiaojing Lai, Bin Chen (University of Hawaii at Manoa); Steve Jacobsen, John Lazarz, Fei Qin, Xiaobing Liu, Michelle Wenz, James Walsh, Phylindia Gant (Northwestern U); Wenge Yang, Gang Liu, Lingping Kong, Qingyang Hu, Liuxiang Yang, Bing Li, Cheng Ji, Huiyang Gou, (Carnegie Institution of Washington); Jiuhua Chen, Fei Han, Xue Liang (FIU); Sean Shieh, ChinPao Wang (U Western Ontario); Feng Zhu, Junjie Dong (U Michigan); Jiachao Liu (Michigan State U); Lu Ma (Ohio State); Zhen Chen,  Guoqiang Tan, Xuanxuan Bi, Tao Zhang, Suoying Zhang, (ANL); Yuejian Wang, Christopher Knill (Oakland U); Arthur Liu, Lisa Liu (Harbin Inst. Tech); Fuyang Liu, Su-Ying Chien, Shengyi Xie, Mingqiang Hou, Bingmin Yan, Bo Gao, Xin Li, Xiaoling Zhou, Biao Wan, Bin Yang,  Cuiying Pei, Xuehui Wei, Li Zhang, Chuanlong Lin, Qian Zhang, Shengyi Xie, Lijuan Wang, Haiyan Zheng, Yajie Wang, Bihan Wang (HPSTAR) Yan Li (Jilin U) Dawei Fan (IGCAS), Oliver Tschauner, Yonggang Wang (UNLV), Jacob Overgaard (Aarhus U), Peter Lazor (Uppsala), Yuanyuan Guo (Nanyang Tech), Jin Zhang (UNM), Yuan Lin, Jin Liu, Qingyang Hu, Samuei Girdzis (Stanford) Jongjin Kim (Yonsei U), Shanti Deemyad, Weizhao Cai, Xiangyi Luo, Rong Zhang (Utah) Changzeng Fan (Yanshan U)

Details of beamtime allocation are listed in Appendix I.

Beamline Operations

Describe beamline operations in the past year. Include the amount of beamtime available; the relationship between COMPRES and the synchrotron facility regarding beamline operations; beamtime proposals and other beamline access; and other relevant discussion.

COMPRES requires also the following specific statistics for the past two years for each beamline:

Number of beamtime proposals received: 61

Number of beamtime proposals granted beamtime: 57

Total number of shifts requested: 467

Total number of shifts granted: 362 (GUP) + 72 (DAC commissioning & setup switch)

Total number of shifts available: 434

Oversubscription rate (= shifts requested / shifts available): 1.08

Number of visits by distinct research groups: 57

Number of unique users,

Categorized by affiliation:

University: 51

Gov’t Lab/agency: 28

Private institution: 7

Categorized by origin

USA: 49

Canada: 2

China: 31

Sweden: 1

Denmark: 1

Korea: 1

Singapore: 1

Total number of person-visits: 251

Number of undergraduate users: 2

Number of graduate student users: 22

Number of visits funded by each funding agency

NSF: 37

DOE: 9

NASA: 2

NNSA: 1

NSERC-Canada: 1

NSF-China: 3

Michigan State: 1

University: 2

HPSTAR-China: 11

Foreign (unspecified): 3

Performance Metrics

General User Time for COMPRES (For higher-ranked proposals with beamtime allocated by the facility): 282 shifts

Contributing User Time (Allocated for COMPRES users who were not assigned GU time): 3 shifts

Total time for COMPRES users: 285 shifts

Beamline development

- Laser heating and black-body temperature readout setup: Contemporary research on Earth-related materials requires not only high pressure, but also high temperature, and laser-heating technique has been broadly used in recent mineralogy experiments. The remotely-controllable laser-heating and blackbody radiation temperature reading setup has been installed in the experimental station 13-BM-C. With the support from COMPRES, a new 200W 1064 nm CW fiber laser has been purchased  and is being integrated to the existing optical platform. The setup is capable of heating the samples up to 3000 K. Preliminary tests on different samples, including hematite, tungsten and platinum have been carried out.

Figure 1: Preliminary laser heating tests. A) hematite sample before heating. B) hematite sample during heating. C) single crystal diffraction pattern of platinum at the melting point (2041 K).

- New detector commissioning: Area detector is one of the most essential components for single crystal diffraction. PX^2 has been using a 15-year old MAR165 CCD detector, and the readout rate is 0.3 frame/s. During the 2017-2 run,  we experienced technical problems with this aging detector (loss of vacuum), which affected the user program. In the summer of 2017, we loaned a new time-resolved Photon II area detector from Bruker. This detector is designed to operate in shutterless mode and features a fast readout rate up to 70 frames/s. Dr. Mark Rivers from GSECARS developed the EPICS driver for this detector.

Figure 2: Test of the Bruker Photon II detector at PX^2.

Beamline community activities

- In December, 2016, PI Przemyslaw Dera and Beamline Scientist Dongzhou Zhang attended the 2016 AGU Fall Meeting in San Francisco, California. Dongzhou presented a poster about the development of the PX^2 program.

- In May 2017, Beamline Scientist Dongzhou Zhang attended the APS user meeting in Argonne, Illinois. Dongzhou gave a poster about the research development of PX^2 program.

- In July 2017, PI Przemyslaw Dera attended the COMPRES Annual Meeting in Tamaya, New Mexico. Przemyslaw Dera gave a talk about the development of PX^2 program. In addition, Beamline Scientist Dongzhou Zhang gave a poster to show the progress of the beamline development.

- In September 2017, PI Przemyslaw Dera attended the EHPRG meeting in Poznan, Poland. Przemyslaw Dera gave a plenary talk which described some of the developments at PX^2.

2018-2019 Budget justification

1. Personnel

The PX^2 Beamline Scientist‘s projected 1.0 FTE salary for 2018 is $74,635. The GLF/PX^2 Scientist‘s projected 0.5 FTE salary for 2018 is $36,414. The anticipated inflation-based increase of base salary is 3% annually. Commissioning and instrument improvement activities will continue at PX^2 through the whole COMPRES IV period. We would like to continue the arrangement with the discounted services of single SOEST mechanical engineer, M. Williamson (half of the support that was required during the initial intensive commissioning of the program). The cost of this support is calculated based on a discounted monthly 1.0 FTE salary of $ $9,291 and 0.1 FTE per month.

2. Fringe Benefits

Fringe benefits for the PX^2 Beamline Scientist and GLF/PX^2 are calculated based on the University of Hawaii at Manoa rate of 58.13%. Fringe benefits for the engineering staff are calculated at 29.11%.

3. Travel Expenses

The PI anticipates requiring travel expense support for the PX^2-related activities at Argonne 2 times a year, with estimated cost of the trip at $2,500. For the PX^2 Beamline Scientist we request funds to travel to the COMPRES Annual Meeting and the AGU Annual meeting. Estimated cost of each trips are $1,500 and $2000, respectively.

4. Materials and Supplies

Diamond anvils and support plates for high pressure experiments			$2,000

Chemicals, heating wires, gaskets, materials for custom parts machining		$1,000

Electronic components, 								$2,000

PDF-4+ database license								$6,000

One of the major expenses in high-pressure experiments is the cost of diamond anvils (~$1K/piece). A budget of $2K is requested to replace broken diamond anvils and perform necessary repairs of damaged anvils.

Any DAC experiments will require supplies such as commercially available chemicals as samples or substrates for sample synthesis, solvents, rhenium gaskets for diamond anvil cells, raw materials for machining of custom parts and adapters, etc. The facility commonly makes these materials available to users. A budget of $1K is requested for these purposes.

PX^2 is a very complex hardware installation featuring over a hundred different motors, detectors of various type and signal processing electronics. These components ware out with time and require updates/upgrades to maintain the state-of-the art. The beamline operation also relies on a variety of software, some of which is commercial. A budget of $2K is requested for these purposes.

We are requesting $6K for purchase of single-seat license for the PDF-4+ crystal structure database. Identification of new phases produced in high pressure and temperature experiments is one of the most important and most challenging tasks involved in analysis of synchrotron mineral physics experimental results. Availability of reliable and comprehensive database of reference crystal structures and reference experimental patterns, such as the commercial PDF-4+ database will greatly help the users of PX^2 facility and other GSECARS beamlines to get the most of their data and streamline interpretation of experimental results. PDF-4+ is the most comprehensive database of this type available on the market. GSECARS already has licenses for commercial software that searches the PDF database (e.g. Jade and Match!), but a current version of the structural database is not available.

5. Indirect Costs

Indirect costs are calculated based on the University of Hawaii approved off-campus rate of 24%.