CDN/H(21)012

Issue 1: 19/08/2021

UNCONTROLLED WHEN PRINTED

Tspd-adq : Software Maintenance Guide

Author: Peter Heesterman

Abstract

This document describes maintenance and install procedure of the Tspd-adq EPICS NDS3 application. It was originally developed by CosyLab for Teledyne to control and acquire data from ADQ digitiser modules. It has been extended to support the ADQ8 module type used by HRTS.

| R.O.: Peter Heesterman | Reviewers: Edward Jones; David Kinna | Approver: John Waterhouse |
| --- | --- | --- |

Distribution

CODAS&IT

Table of Contents

1.	Purpose	3

2.	Scope	3

3.	Introduction	3

4.	Notes	3

4.1	Source Control	3

4.2	Windows build options	3

5.	Operating system	3

6.	Installation	4

6.1	Hardware	4

6.2	Software	4

6.3	Firmware upgrade.	4

7.	IOC start-up	5

8.	Installation verification	5

9.	Development Environment	8

10.	Creating a release	8

11.	Revision history	9

12.	References	9

# Purpose

The document is mainly focused on usage at JET and MAST. Usage elsewhere may be different, but this document should still provide a helpful guide.

This document has been written after deployment of the software at JET and MAST.

# Scope

This document contains information on the structure and operation of the tspd-adq project. It is intended for developers who need to maintain the application.

# Introduction

The tspd-adq software provides control, monitoring and data acquisition from Teledyne ADQ digitiser modules.

In principle, the software should operate with any ADQ module type, but the author has tested this only with the ADQ8 modules which are used by the JET HRTS project and the ADQ14 modules that are used by the MAST XTS project.

# Notes

## Source Control

Work in progress is stored in the CCFE GIT repository at https://git.ccfe.ac.uk/DATAnet/tspd-adq.

This is a sub-module of https://git.ccfe.ac.uk/DATAnet/epics.git and is also dependent on https://git.ccfe.ac.uk/DATAnet/nds3_epics.

These are available for access outside of CCFE at:

Tspd-adq -  TBC?

NDS3 - https://github.com/Cosylab/nds3 and https://github.com/Cosylab/nds3_epics

## Windows build options

For Windows users only, there are two different possible methodologies.

- The code can be built using the EPICS make system, as would be the case on other operating systems.

- The code can be built using Visual Studio project files.

# Operating system

The software has been tested on Windows 10 LTSB in accordance with [O/S Policy]. It has also been tested with Centos 8.

# Installation

## Hardware

The JET HRTS project is installed with 12 Teledyne ADQ8 modules that are housed in two NI PXIe-1084 chasses. These are inter-connected by means of NI PXIe-8374 cards and are connected to the PC by means of a PCIe-8375 PXI remote control card.

The MAST XTS project is installed with 12 Teledyne ADQ14 modules that are housed in one ADLINK PXES-2780 chassis holding an ADLINK PXIe-3985 embedded PC.

## Software

[ADQAPI] needs to be installed.

The Windows installer is available at TSPD-SDK-installer_r57945.exe. The Linux installer is also present in that folder.

It is required that the installed version number is the same as that used for building the code, this is enforced by software start-up.

JET HRTS (using National Instruments PXIe hardware) requires:

- NI PXI Platform services 18.5 – for the PXIe card driver

  - Runtime support

  - Configuration support

- Configuration and Utilities

  - NI Measurement and Automation Explorer (MAX) 18.5

MAST XTS (using ADLink PXIe hardware) requires:

- PXES-2780, drivers for the chassis.

- PXIe-3985, motherboard drivers for the embedded PC.

- ADL-GPIB, GPIB drivers for the embedded PC.

- ADLINK_MAPS, device management and configuration utility.

- ADLINK_APPS, PXI platform services

These are all available at \\catsserver.jetcats.jet.efda.org\Downloads\ADLink .

## Firmware upgrade.

The ADQ14 modules firmware was upgraded using the ADQAssist utility that is installed as part of the [ADQAPI] install.

The firmware file is available as ADQ14_4A_FWDAQ_r56485.mcs.

# IOC start-up

The IOC is normally started by means of a batch file, which invokes the appropriate st.cmd start-up file.

It also redirects (stdout and stderr) output to a log file; this is helpful in diagnosing fault conditions. The file may be viewed using e.g. Notepad++.

- st-multi.bat for JET HRTS starts the IOC using st-multi.cmd.

- st-MAST.bat for MAST XTS starts the IOC using st-MAST.cmd.

# Installation verification

When the software is started, the IOC should launch and successfully connect with each of the modules. If this is not the case, the IOC log will show many repeated error messages.

A normal start-up will look like:

#!../../bin/linux-x86_64/adq-test

< envPaths

cd "C:/tspd-adq"

## Register all support components

dbLoadDatabase "dbd/adq-test.dbd"

adq_test_registerRecordDeviceDriver pdbbase

Warning: IOC is booting with TOP = ""C:/tspd-adq""

but was built with TOP = "D:/epics/iocTops/tspd-adq/"

## Load library of ADQ module

# LoadDriver required for Linux

# ndsLoadDriver("${ADQ_LIB}/libtspd-adq.so")

# Or for Windows

ndsLoadDriver("tspd-adq.dll")

ndsLoadNamingRules ("C:/tspd-adq/tspd-adqApp/src/ADQDeviceNamingRules.ini")

epicsEnvSet("ROOT", "ADQ")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06905")

epicsEnvSet("DIGITNO", "A_")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-A_, ADQSN = 06905")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-A_", "ADQSN=06905")

## Enable info logging in NDS, needs a rootNode name

# nds setLogLevelInfo "$(ROOT)-$(DIGITNO)"

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06906")

epicsEnvSet("DIGITNO", "CM")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-CM, ADQSN = 06906")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-CM", "ADQSN=06906")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06907")

epicsEnvSet("DIGITNO", "EO")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-EO, ADQSN = 06907")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-EO", "ADQSN=06907")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06908")

epicsEnvSet("DIGITNO", "GQ")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-GQ, ADQSN = 06908")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-GQ", "ADQSN=06908")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06909")

epicsEnvSet("DIGITNO", "IS")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-IS, ADQSN = 06909")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-IS", "ADQSN=06909")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06910")

epicsEnvSet("DIGITNO", "KU")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-KU, ADQSN = 06910")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-KU", "ADQSN=06910")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06911")

epicsEnvSet("DIGITNO", "V_")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-V_, ADQSN = 06911")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-V_", "ADQSN=06911")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06912")

epicsEnvSet("DIGITNO", "BL")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-BL, ADQSN = 06912")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO namStarting iocInit

iocRun: All initialization complete

e), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-BL", "ADQSN=06912")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06913")

epicsEnvSet("DIGITNO", "DN")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-DN, ADQSN = 06913")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-DN", "ADQSN=06913")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06914")

epicsEnvSet("DIGITNO", "FP")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-FP, ADQSN = 06914")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-FP", "ADQSN=06914")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06915")

epicsEnvSet("DIGITNO", "HR")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-HR, ADQSN = 06915")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-HR", "ADQSN=06915")

### ----- Do this for each digitizer, changing the ROOT each time

# Define serial number of the digitizer

epicsEnvSet("ADQSN", "06916")

epicsEnvSet("DIGITNO", "JT")

## Load record instances

dbLoadTemplate("db/ADQDevice.substitutions8", "PREFIX = ADQ-JT, ADQSN = 06916")

## Create device with parameters: driver name (from NDS_DEFINE_DRIVER, not .SO name), rootNode, digitizer's serial number

ndsCreateDevice(tspd_adq, "ADQ-JT", "ADQSN=06916")

### -----

cd "C:/tspd-adq/iocBoot/iocadq-test"

iocInit

############################################################################

## EPICS R7.0.3.1

## EPICS Base built Aug  4 2021

############################################################################

epics>

# Development Environment

The system has been built on Windows using the following tools:

- Microsoft Developer Studio 2017.

- ActiveState Perl 5.26.0.2600.

- GNU Win32 Make.

# Creating a release

For JET use, buildlogs\makerelease.bat copies all the appropriate files to the \\CatsServer.JetCats.Jet.Efda.Org\CopyArea\HRTS\tspd-adq directory.

This directory may then be copied on the operational PC as C:\Tspd-adq.

# Revision history

| Issue | Author | Date | Description |
| --- | --- | --- | --- |
| 1 | P. Heesterman | 19/08/2021 | First issue |

# References

| Ref. | Document | Date | Details and version |
| --- | --- | --- | --- |
| ADQAPI | ADQAPI Reference Guide | Mar 2021 | 57946 |
| O/S Policy | CDN/H(O9)005 | 30/08/2019 | 5 |