Diane Fairley

11/11/08

Device ‘Access’ PV for HLAs

Requirements

# Scope

High level software applications generally perform ‘automated’ functions for physicists and operators - giving the user simple, button-push command of complicated control and analysis operations.   An application can quickly and automatically move motors, scan through magnet or klystron settings, take data and analyze it with no operator intervention.  But there are times when devices do not perform as required. At such times, it can save the user time and effort if the operator has a way of telling software applications not to use a device that is not functioning properly. A ‘access’ PV for each readable and/or controllable device can give the operator the ability to set the status of the device for application software.

# Background and examples

There may be times when a device such as a BPM is producing TMIT readings, but those readings are always ‘too low’.  Similarly, a controllable device, such as a motor for a wire, might accept a move command, but not actually move to where it was commanded to go. When an application uses a device, it can report errors such as connection errors when the device is ‘not there’. An application can even report a conflict between a ‘setpoint’ and a ‘readback’, but an application should not decide that the device is not functioning properly.    Instead, it may report the error (if possible) repeatedly, or worse, continue to use the device and produce erroneous results. This can be very frustrating to users, particularly over the many hours or days that a device may remain broken or offline while repairs are being pursued.

The feedback application is the first application to require this type of PV.  Feedbacks run 24/7, and are configured to use a set of measurement and control devices. When one of those devices breaks, the feedback can often continue without the device, or by using a different device

A ‘access’ PV per readable and/or controllable device should be added to the IOC that allows an operator to set an operating ‘state’ for each device.  The requirements for this PV are described below.

# Requirements

- There will be a 'access' pv per device that stores a 'state' of that device as decided by an operator.

- The 'access' PV is settable by an operator, read-only to software applications.

- The 'access' PV is not required to cause an action on the device itself, it merely stores a 'state' for the device.

- The name of the pv attribute is to be determined by the IOC developers group, but should have the macro-ized form of $(DEV)attribute_name_suffix (where attribute_name has previously been ACCESS.) so that it can be instantiated per device.  It is recommended this name be differentiated from the IOC ACCESS pv that all IOCs use currently.

- It is recommended that the 'access' pv have 4 states, which perform as follows (I think this is what the SCP definitions were, but Steph or anyone who knows feel free to correct this ):

- Online - this device is operational and ready to be used by production software

- Maintenance - this device is in a maintenance state, where maintainers can see how it is performing, but is not to be used for production. Maintenance software can use the device.

- Checkout - This device has had maintenance work done and is back in production for 'check out' purposes before saying its good for all production use.  Some software may use it, other applications may not (maybe depending on how critical the the software is to operations? Some apps may be allowed to use 'checkout' devices because it's a good way to check out it's operation?)

- Offline - this device is not to be used by any high level software

- The names of the states are to be determined by the IOC developers group.